From: VTS Committee VTS28

To: LAP

Liaison Note

Draft Guidelines – Provision of Navigational Assistance Service by a Vessel Traffic Service

Introduction

The VTS Committee has been directed to “Review/provide guidance on navigational assistance service.”  In the course of addressing this task the committee identified worrying indications that differing guidance was being given to VTS Operators in the interpretation and implementation of the part of the IMO Resolution A.857(20) relating to “result-oriented instructions.”

The VTS Committee seeks comments from the Legal Advisory Panel to ensure all reference material being used to address this issue is being properly interpreted and applied.

Comments

A draft guideline on navigational assistance has now been prepared by the VTS Committee as directed by Council.  The draft draws on existing regulations and recommendations, specifically:

- SOLAS Regulation V-12 Vessel Traffic Services

- IMO Resolution A.857(20) Guidelines for Vessel Traffic Services

- IMO Resolution A.918(22) IMO Standard Marine Communication Phrases, and

- IALA Vessel Traffic Services Manual, 4th Edition

Furthering work on this issue, however, requires that the committee resolve a considerable difference of opinion regarding whether a VTS may provide a vessel with a course to steer as advice when providing Navigational Assistance.

The VTS Committee is not in full agreement on whether it is considered to be a “result oriented instruction” when a course to be steered is provided as “advice.”  The committee fully agrees that a course to be steered should never be provided as an “instruction.”  The distinction between “advice” and “instruction” is made in the context of their respective definitions as found in the IMO’s Standard Marine Communication Phrases (SMCP).

It is therefore requested that the LAP review and advise the VTS Committee on the guidance proposed in the current draft in order to inform further work.  The VTS Committee is particularly concerned that the guidance be consistent with that already provided by the relevant IMO instruments.

Action Requested

That the LAP provide comment on the issue presented in the draft guidelines on Navigation Assistance Service and the VTS Committee’s interpretation of the guidance regarding provision by a VTS of a course to steer to a vessel as described in IMO Resolution A.857(20) Guidelines for Vessel Traffic Services and IMO Resolution A.918(22) IMO Standard Marine Communication Phrases.

DRAFT

VTS28/WG1/WP04rev2

IALA Guideline No. XXXX

Provision of a Navigational Assistance Service by VTS

Edition 1

September 2008

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Telefax +33 1 34 51 82 05 E-mail : iala-aism@wanadoo.fr	Internet : http://www.iala-aism.org

Table of Contents

1	Overview	1

1.1	Scope	2

2	References	1

3	Definitions	2

4	Abbreviations	4

5	Provision of a Navigational Assistance Service	5

5.1	Responding to traffic situations developing in the VTS area	5

5.2	Interaction between a VTS and a vessel	6

5.2.1	Master / VTS Relationship	6

5.2.2	Occasions for Use of Navigational Assistance	6

5.2.2.1	On Request	6

5.2.2.2	Observed / Deemed Necessary	6

5.2.3	Delivery of Navigational Assistance	7

5.2.3.1	Result Oriented Messages	7

5.2.3.2	Communication Messages and Message Markers	8

5.2.3.3	Information	8

5.2.3.4	Warning	9

5.2.3.5	Advice	10

5.2.3.6	Instruction	10

5.2.3.7	Summary	12

5.3	Other Considerations	12

5.3.1	Equipment Capabilities	12

5.3.2	Staffing and Training	12

5.3.3	Legal	12

5.4	Operational Procedures	12

5.5	Promulgation of Information and Types of Services	13

# Overview

The purpose of vessel traffic services is to improve the safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, worksites and offshore installations from possible adverse effects of maritime traffic.

The principles of vessel traffic services are governed by a hierarchy of regulatory requirements and guidelines.  Key requirements and guidelines include:

- SOLAS Regulation V-12 “Vessel Traffic Services”

- IMO Resolution A.857(20) Guidelines for Vessel Traffic Services

- Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems

- IMO Resolution A918(22) IMO Standard Marine Communication Phrases

- IALA Vessel Traffic Services Manual, 4th Edition

Chapter V “Safety of Navigation” of the SOLAS 1974 Convention, Regulation V-12 “Vessel Traffic Services”, states, amongst other things, that:

- “Vessel traffic services contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.”

- “Contracting Governments planning and implementing VTS shall, wherever possible, follow the guidelines developed by the Organization.”

Implicit in the statement that contracting governments, wherever possible, shall follow guidelines developed by the Organisation, is the need to ensure consistency in the approach taken to the delivery of VTS and to avoid confusion for the mariner trading between various jurisdictions about the delivery of VTS services.

IMO Resolution A.857(20) Guidelines for Vessel Traffic Services define a Vessel Traffic Service (VTS) as a:

Service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

In providing definitions and clarifications with regards to VTS services, IMO Resolution A.857(20) also states that:

“VTS should comprise at least an Information Service and may also include others, such as a Navigational Assistance Service or a Traffic Organisation Service, or both.”

A Navigational Assistance Service may be initiated in response to navigational situations developing in the VTS area.  It is important that assistance to onboard decision making is provided by the VTS in a timely manner, is clearly understood by both parties and is not open to misinterpretation to minimise the risk of unexpected and dangerous reactions.

In providing guidance on the implementation of a Navigational Assistance Service by an authorised VTS the IALA VTS Manual (2008) states that:

A Navigational Assistance Service is “a service to assist in the on-board navigational decision-making process and is provided at the request of a vessel, or when deemed necessary by the VTS. A Navigational Assistance Service provides essential and timely navigational information to assist the on-board decision-making process and may inform, advise and/or instruct vessels accordingly.”

## Scope

The aim of this document is to provide:

- Guidance on the delivery of a Navigational Assistance Service by a VTS to ensure that interaction with participating vessels to assist onboard decision making is consistent between VTS centres, is timely and is clearly understood.  This guidance will be issued as a guideline until the development of the next edition of the VTS Manual.  It will then be considered for incorporation into the appropriate section(s) of the VTS Manual, and

- Clarification to IMO Resolution A.857(20) Guidelines for Vessel Traffic Services,  and IMO Resolution A.918(22) IMO Standard Marine Communication Phrases,

# References

| IMO | SOLAS Regulation V-12 “Vessel Traffic Services” |
| --- | --- |
|  | Resolution A.857(20) Guidelines For Vessel Traffic Services |
|  | Resolution A918(22) IMO Standard Marine Communication Phrases |
| IALA | IALA Recommendation V-127 On Operational Procedures for Vessel Traffic Services |
|  | VTS Manual (2008) 4th Edition |
|  | IALA Recommendation V-128 – Operational and Technical Performance Requirements for VTS Equipment Edition 3.0 June 2007 |
|  | IALA Recommendation V-103 - VTS Operator training |

Capture all references together in one section

# Definitions

The following terms are used in conjunction with vessel traffic services:

| Advice |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to influence others by a recommendation.”2 Note: The decision whether to follow the ADVICE remains with the recipient. ADVICE does not necessarily have to be followed but should be considered very carefully. |
| --- | --- | --- |
| Competent Authority |  | The authority made responsible, in whole or in part, by the Government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment. 1 |
| Information |  | This indicates that the interaction from a VTS Centre “is restricted to observed facts, situations” 2 It does not involve any interpretation, evaluation or judgment on the part of the VTS operator. Note: This is preferably used for navigational and traffic information. It is the recipient’s responsibility to incorporate it into their navigational decision making and to decide the appropriate action to take in response to the INFORMATION provided. |
| Instruction |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to influence others by a regulation.” 2 Note: This means that the sender must have full authority to send such a message. The recipient has to follow this legally binding message unless he/she has contradictory safety reasons which then have to be reported to the sender Whilst the responsibility must always remain with the master of a vessel, a decision to ignore an INSTRUCTION may have to be fully justified. |
| Participating Vessel |  | Resolution A.857(20) states that “Vessels navigating in an area where vessel traffic services are provided should make use of these services. Depending upon governing rules and regulations, participation in a VTS may be either voluntary or mandatory. Vessels should be allowed to use a VTS where mandatory participation is not required.” |
| Vessel Traffic Services (VTS) |  | a service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.1 VTS should comprise at least an information service and may also include others, such as a navigational assistance service or a traffic organization service, or both.1 |
| VTS Authority |  | The authority with responsibility for the management, operation and coordination of the VTS, interaction with participating vessels and the safe and effective provision of the service. 1 |
| VTS Operator |  | An appropriately qualified person performing one or more tasks contributing to the services of the VTS. 1 |
| VTS traffic image |  | The surface picture of vessels and their movements in a VTS area. 1 |
| VTS Centre |  | The centre from which the VTS is operated. 1 |
| Warning |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to inform others about danger.” 2 |
| 1 | IMO Resolution A.857(20) Guidelines For Vessel Traffic Services |  |
| 2 | IMO Resolution A.918(22) IMO Standard Marine Communication Phrases |  |

# Abbreviations

| IALA | International Association for Marine Aids to Navigation and Lighthouse Authorities |
| --- | --- |
| IMO | International Maritime Organization |
| MSC | Maritime Safety Committee (Standing Committee of IMO) |
| VTS | Vessel Traffic Services |
| SOLAS | Convention on the Safety of Life at Sea |
| SMCP | Standard Marine Communication Phrases |
| VTSO | Vessel Traffic Services Operator |
| OOW | Officer of the Watch |
| SOG | Speed over Ground |
| COG | Course over Ground |

# Provision of a Navigational Assistance Service

A Navigational Assistance Service is a service that provides essential and timely navigational information to assist in the onboard navigational decision-making process and to monitor its effects. It may also involve the provision of navigational advice and/or instruction.

Navigational Assistance may be provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary. Such assistance requires positive identification and continuous communication throughout the process.

It is important to recognise that a navigational assistance service:

- Is a service provided to an individual vessel, at the request of the vessel or when deemed necessary by the VTS, to assist the decision making process on board the vessel concerned. This service consists of navigational matters relating to a specific vessel and may include warnings, advice and instruction subject to the authority of the VTS.

- Is a service that has a start and end time

A Navigational Assistance Service may be initiated in response to traffic and navigational safety situations developing in the VTS area.  It is important that information to assist the onboard decision making is provided in a timely manner.  It should be clearly understood by both parties and is not open to misinterpretation to minimise the risk of unexpected and dangerous reactions.

## Responding to traffic situations developing in the VTS area

IMO Resolution A.857(20) states that:

A VTS should at all times be capable of generating a comprehensive overview of the traffic in its service area combined with all traffic influencing factors.

The VTS should be able to compile a traffic image, which is the basis for its capability to respond to traffic situations developing in its service area. The VTS traffic image allows the VTS operator to evaluate situations and make decisions accordingly.

To respond to traffic situations developing in the VTS area and to decide upon appropriate actions the acquired data should be processed and evaluated. Conclusions from the evaluation need to be communicated to participating vessels.

Examples of developing situations where Navigational Assistance may be requested or deemed necessary by the VTS include:

- Risk of grounding

- Vessel deviating from the recommended track

- Vessel unsure of its position or unable to determine its position

- Vessel unsure of the route to its destination

- Assistance to a vessel to an anchoring position

- Vessel navigational or manoeuvring equipment casualty

- Extreme environmental conditions (e.g., low visibility, high winds)

- Potential collision between vessels

- Potential collision with hazard

## Interaction between a VTS and a vessel

### Master / VTS Relationship

Where a VTS provides navigational assistance, irrespective of whether the service is initiated by the VTS or the participating vessel, care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot (IMO Resolution A.857(20) Guidelines For Vessel Traffic Services).

Further, masters of vessels and pilots should aware of their responsibilities in receiving communications using IMO Resolution A.918(22) IMO Standard Marine Communication Phrases (SMCP) message markers.

Decisions concerning the actual navigation and the manoeuvring of the vessel remain with the master. Neither a VTS sailing plan, nor requested or agreed changes to the sailing plan can supersede the decisions of the master concerning the actual navigation and manoeuvring of the vessel.

### Occasions for Use of Navigational Assistance

As has been noted, Navigational Assistance is a service that is initiated at the request of a participating vessel or when deemed necessary by the VTS.

#### On Request

Navigational Assistance may be provided on request by a vessel in circumstances such as equipment failure or navigational unfamiliarity.  Individual circumstances will dictate the degree of preparation that can be undertaken prior to commencing a Navigational Assistance Service on Request but, wherever possible preparations should include an assessment of the capability of the vessel to undertake the passage safely and/or the risks involved in declining assistance if the request is sought after the vessel has entered restricted waters.

The provision of Navigational Assistance does not absolve the master from his responsibility for the safety of his own ship; in particular the Master should be made aware of the limitations on VTS equipment and the specific responsibilities of the Master for collision avoidance.

If possible and if time permits, checks should normally be made prior to commencement of the provision of Navigational Assistance to assess the capability of the vessel to respond to the guidance given.  An example checklist is at Annex 1, which should be modified as required for local requirements.

#### Observed / Deemed Necessary

Navigational Assistance may be provided when the VTS observes a developing situation (e.g a vessel deviating from a recommended route) and deems it necessary that interaction with the master/OOW is necessary.

Under such circumstances, it is likely that the immediate priority will be placed on providing the necessary assistance before attempting to formally negotiate the commencement of Navigational Assistance.  However, once the immediate situation has been resolved, the continuation or completion of Navigational Assistance should be subsequently clarified and the use of the check list considered.

### Delivery of Navigational Assistance

In providing navigational assistance it is important that the interaction from a VTS centre to assist the onboard decision making is conducted in a timely manner, is unambiguous and clearly understood by both parties and is not open to misinterpretation. In providing guidance for the delivery of VTS services IMO Resolution A.857(20) Guidelines for Vessel Traffic Services recommends that in the provision of information, advice or instruction:

- IMO Standard Marine Communication Phrases should be used where practicable (IMO Resolution A.918(22) IMO Standard Marine Communication Phrases).

- Messages relating to Navigational Assistance must always be addressed by name to the vessel participating in Navigational Assistance so that there is no doubt to whom the content of the message is directed.

- Consideration should be given regarding the VHF radio frequency on which the Navigational Assistance Service should be provided.  The decision should depend on individual and local circumstances.  However, an assessment should be made of the relative benefits of conducting the assistance on a discrete frequency so that interference from other users can be avoided, or the use of  a common working frequency such that other users are aware of the likely actions of the vessel participating in the Navigational Assistance Service.  Other options may be available if the participating vessel is able to monitor two or more frequencies.

In providing essential and timely navigational information to assist the onboard decision-making process a VTS may provide:

- Information

- Warning

- Advice

- Instruction

IMO Resolution A.857(20) Guidelines For Vessel Traffic Services states that in the interaction between a participating vessel and a VTS centre:

- “A distinction should be made between the provision of navigational information, being a relay of information extracted from the VTS sensors and the traffic image, and the provision of navigational advice, where a professional opinion is included”, and

- “In any VTS message directed to a vessel or vessels it should be made clear whether the message contains information, advice, warning, or an instruction”, and

- “IMO Standard Communication Phrases should be used where practicable.”

#### Result Oriented Messages

A fundamental principle of VTS communications is that, when authorised to provide Navigational Assistance to vessels, any instruction communicated should be result-oriented only; leaving the details of execution, such as course to be steered or engine manoeuvres to be executed, to the master or pilot on board the vessel..

The interpretation of “Result Oriented” will depend on the situation and context. Phrases that are used for vessel conning in particular specific rudder or engine commands, such as “Stop Engine” or “Hard to Starboard” should never be used.  If it is considered appropriate to issue guidance on courses to be steered or speed, then it is important that it is made clear that such guidance is given as a recommendation and that the decision whether to follow the ADVICE remains with the recipient.  Use of the ADVICE message marker should be considered to ensure that there is no doubt regarding where the final responsibility lies for acting on the guidance given.

#### Communications Using Message Markers

There are eight types of Communication Message Markers that are frequently used in VTS  which may be used to emphasise the content of the message or to ensure that the message will be properly understood, particularly when language difficulties are apparent between the VTS and the vessel.  These are:

- Information

- Warning

- Advice

- Instruction

- Question

- Answer

- Request

- Intention

For more information see the IMO Resolution A.918(22) IMO Standard Marine Communication Phrases.

It is at the discretion of the shore personnel or the ship's officer whether to use one of the message markers and, if so, which marker is applicable to the situation. However, since the provision of Navigational Assistance is likely to have arisen out of some form of shortcoming, failure or emergency; the contents of all messages directed to a vessel should be clear.  It is therefore recommended that Message Markers are always used in the delivery of a Navigational Assistance Service.  If used, the message marker is to precede the message or the corresponding part of the message.

Examples of the use of the message markers QUESTION, ANSWER, REQUEST and INTENTION are provided in SMCP.  The use of the message markers INFORMATION, WARNING, ADVICE and INSTRUCTION are particularly relevant to the provision of a Navigational Assistance Service and are further explained below.

#### Information

SMCP defines “Information as communication whereby the message is restricted to observed facts, situations, etc. and is preferably used for navigational and traffic information, etc. “

As such, it is a relay of information extracted from the VTS sensors and the traffic image where no professional opinion by the VTSO is included, other than the determination by the VTSO that the information is relevant to the mariner.

Implicit in this definition is that the consequences of using the INFORMATION will be up to the recipient.

Examples of the provision of an Information Message during the delivery of Navigation Assistance to a vessel include:

- Course and speed over the ground by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards; and

- Positions, identities, intentions and any restrictions of surrounding traffic.

For example:

| Warning Example 1 | “INFORMATION, According to my equipment , vessel “No Name” will overtake you in the vicinity of Buoy “XXXX”. |
| --- | --- |
| Example 2 | “INFORMATION, Next high water at Port “YY” predicted to be “XX:XX” at a height of “ZZ” metres. |

The provision of information during the delivery of Navigation Assistance may also include Warnings.  SMCP defines WARNING as communication whereby the message implies “the intention of the sender to inform others about danger”.  It may be used to convey potentially dangerous situations or observed developing situations.

As such, it is a relay of information extracted from the VTS sensors and the traffic image and, in the professional opinion of the VTSO, the message should be communicated to inform a vessel about potential danger.

The contents of a Warning Message should be immediately assessed onboard the vessel in conjunction with any additional information which may not be available to the VTS centre and corrective action taken where necessary.  Subject to response to the vessel, a Warning Message may be followed by further messages such as ADVICE or INSTRUCTION

Implicit in this definition is that the recipient should pay immediate attention to the danger mentioned.  The provision of further messages such as ADVICE or INSTRUCTION may assist the recipient in assessing the consequences of using the information provided.  Examples of the provision of a Warning Message during the delivery of Navigation Assistance to a participating vessel include:

| Example 1 | "WARNING. Obstruction in the fairway. Submerged container …. degrees, distance XX meters from ?? Buoy.” |
| --- | --- |
| Example 2 | “WARNING”. According to my equipment you are running into shallow water,.” |
| Example 3 | WARNING. According to my equipment, your course is close to the outgoing vessel bearing …… degrees distance … nautical miles |

#### Advice

SMCP defines ADVICE as communication whereby the message “implies the intention of the sender to influence the recipient by a recommendation.”

Implicit in this definition is:

- A professional opinion on the part of the VTSO is included in the message as a means to influence the recipient, and

- The recipient should pay immediate attention to the advice mentioned and the consequences of using the information provided will be up to the recipient.  Advice does not necessarily have to be followed but should be considered very carefully by the recipient.

- The recipient should always inform the VTS of its intentions.

The provision of advice in response to a developing situation may also include or require:

- An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- An assessment of the environmental conditions;

- An assessment of the implications of the cargo carried;

Examples of the provision of ADVICE during the delivery of Navigation Assistance, to a participating vessel include:

| Example 1. | “WARNING. According to my equipment, you are diverting from the recommended track. “ADVICE, Follow the recommended track. |
| --- | --- |
| Example 2 | ADVICE. Recommend course over the ground ….. degrees. |
| Example 3 | “WARNING. According to my equipment you are running into shallow water, distance X.X .” “ADVICE. Recommend you steer a new course of …… degrees.” |

SMCP states that VTS personnel and mariners should be fully aware of the implications of words such as "track", "heading" and "course made good".  When used in a VTS during NAS this should be understood as:

Track - The path followed, or to be followed, between one position and another

Heading - The horizontal direction of the vessel's bows at a given moment measured in degrees clockwise from north.  Heading is normally expressed as a recommendation to steer a new course and should only be used as a measure of last resort.

Course made good - That course which a vessel makes good over ground, after allowing for the effect of currents, tidal streams, and leeway caused by wind and sea.  In a VTS this is often expressed as “Course Over Ground”

#### Instruction

SMCP defines INSTRUCTION as communication whereby the message implies the intention of the sender to “influence the recipient by a Regulation”.

Implicit in this definition is:

- The message is a direction given by the VTS centre under the provisions of a statutory authority.

- The sender must have delegated authority to communicate such a message.

- The recipient has a legal obligation to comply with this message unless contradictory safety reasons exist, which then have to be reported immediately to the sender.

The use of INSTRUCTION as part of Navigational Assistance is an unusual circumstance that well may be an action of last resort and is likely to require local authorisation.

IMO Resolution A.857(20) Guidelines for Vessel Traffic Services states that:

When the VTS is authorized to issue instructions to vessels, these instructions should be result-oriented only, leaving the details of execution, such as course to be steered or engine manoeuvres to be executed, to the master or pilot on board the vessel. Care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot

Thus, whilst it is acceptable to issue ADVICE on heading and speed, it would not be appropriate to issue heading and speed requirements as an INSTRUCTION.

Generally masters of vessels will respond promptly and carry out instructions given by a VTS. However, it should be recognised that there may be occasions when an instruction by a VTS is disregarded because the master has additional information not available to the VTS Centre and he decides on another course of action. For example, a vessel that is not being tracked by the VTS may be a contributing factor to the navigational situation.

Even when there are no apparent language difficulties between the VTS and the vessel, the use of the INSTRUCTION message marker is recommended to reinforce the content of the transmission.

Examples of the provision of Instruction during the delivery of Navigation Assistance, to a participating vessel include:

| Example 1 | "WARNING There is a restricted area South of you distance 1.2 nautical miles" “INSTRUCTION. Do not enter this area” |
| --- | --- |
| Example 2 | “WARNING. Visibility in the approach channel is less than …... nautical miles. “INSTRUCTION. Do not enter the channel” |

#### Summary

The table below summarises the message markers that should be adopted with the respective category of service provided when it is assessed that SMCP should be used.

| Interaction | Type of Service |  |  |
| --- | --- | --- | --- |
|  | Information Service | Traffic Organisation Service | Navigational Assistance Service |
| Information | Y | Y | Y |
| Warning | Y | Y | Y |
| Advice |  | Y | Y |
| Instruction |  | Y | Y1 |

1The use of INSTRUCTION as part of Navigational Assistance is an unusual circumstance that well may be an action of last resort and is likely to require local authorisation.

## Other Considerations

In determining the provision of a Navigational Assistance Service there is a need to give emphasis to equipment capabilities, staffing and training and the legal basis.

### Equipment Capabilities

Consideration should be given to the quality of the surface picture available, the communications capability and the equipment availability in determining to provide Navigational Assistance.  For further information refer to IALA Recommendation V-128 – Operational and Technical Performance Requirements for VTS Equipment Edition 3.0 June 2007.

### Staffing and Training

Consideration should be given to the skills and training of the VTSOs in determining to provide Navigational Assistance.

The authorisation of VTS personnel to provide this service should also be identified. VTS Authorities should give careful consideration to shore staffing levels and the qualifications of VTS personnel.

For further information refer to IALA Staffing Level Guidelines as well as the V-103 documentation.

### Legal

Consideration should be given to the national and international legal basis for the provision of a Navigational Assistance Service.

## Operational Procedures

In providing Navigational Assistance clear operational procedures should be in place.

For further information and guidance on preparing operational procedures refer to the example checklist at Annex 1 of this guideline and IALA Recommendation V-127 On Operational Procedures for Vessel Traffic Services.

## Promulgation of Information and Types of Services

The services offered to the mariner by a VTS should be promulgated to vessels in the appropriate internationally recognised marine publications, including the World VTS Guide. This should include details of the VTS, its capabilities, rules, regulations, requirements and procedures. The information promulgated should be verified and up-dated at least at annual intervals.

Where a VTS has lost the capability to provide Navigational Assistance this should be promulgated in a timely manner.

Annex 1

EXAMPLE1

VTS CHECKLIST FOR THE PROVISION OF NAVIGATIONAL ASSISTANCE

VTS:- …………………

VESSEL:-……………………………	MASTER’S NAME:-…………………

Review:

- Positive Identification

- Vessel’s Machinery State

- Vessel’s Navigational equipment State

- Up-to-Date charts

- Master/Officer knowledge of English/Local language

- Master’s knowledge of Local Area

- Environmental Conditions (Wind, Day/Night, Visibility, Tidal Height, Tidal Stream)

- Traffic Levels

- Cargo

- VTS Operator Competence/Authorisation

- VTS Equipment performance, serviceability and back-up (particularly key elements of communications, radar and AIS)

Assess and Decide:

- Capability of vessel to continue passage under VTS NAS.

- The risk of not providing NAS if the vessel is already in restricted waters.

- Need for amendment to passage plan (such as a temporary anchorage).

- Alternative options such as provision of a pilot.

- Communications channel for provision of NAS.

Agree:-

  - Master’s acceptance to continue passage under Navigational Assistance.

  - Master’s understanding that NAS is advisory and does not in any way absolve him from his responsibility for the safety of his vessel or for Collision Avoidance.

  - Commencement of NAS

  - Completion of NAS

  - VHF Channel

1 This checklist is an example, which should be modified and amplified by operational procedures as required for local requirements through risk assessment.

Resolution A.857(20) states that “Vessels navigating in an area where vessel traffic services are provided should make use of these services. Depending upon governing rules and regulations, participation in a VTS may be either voluntary or mandatory. Vessels should be allowed to use a VTS where mandatory participation is not required”

A WARNING message is a particular type of  SMCP communication message that may be appropriate for use in drawing attention to the significance of a message providing “information” during Navigational Assistance Service as described in the IALA VTS Manual.