VTS28/14/1

also e-NAV5/7/4

September 8-12, 2008

Submitted by the European Maritime Safety Agency (EMSA)

Sharing of marine traffic information in the European Union through SafeSeaNet

1. SafeSeaNet background

Following the ERIKA accident off the French coast in 1999, the European Union (EU) adopted several legal instruments for improving the prevention of accidents at sea and combating marine pollution. Directive 2002/59/EC aims at establishing in the Community a vessel traffic monitoring and information system “with a view to enhancing the safety and efficiency of maritime traffic, improving the response of authorities to incidents, accidents or potentially dangerous situations at sea, including search and rescue operations and contributing to a better prevention and detection of pollution by ships”.

To achieve these objectives, the European Commission launched the development of a European data network, the system now called SafeSeaNet. The main objective of SafeSeaNet is to provide a European platform for data exchange between the maritime administrations.

Implementation of Directive 2002/59/EC, as well as provisions of other instruments of European legislation, requires the collection and distribution of various kinds of data. The current SafeSeaNet works in a notify, request and response mode.

In accordance with the Directive 2002/59/EC, Member States shall notify (within specified time limits) the following information to the European Index Server (EIS):

- Port Notifications (identifying the destination and ETA prior to arrival)

- Hazmat Notifications (for ships with dangerous or polluting goods – SOLAS/MARPOL)

- Ship Notifications (AIS and Mandatory Reporting Systems information). VTSs and MRS receive and exchange similar information on ships operating within their areas.

- Alert Notifications (messages issued by coastal stations following events at sea e.g. accident or incident, Search and Rescue report, Pollution report, Waste alert notification, Lost/Found container, non-compliance with Directive requirements etc.)

SafeSeaNet is built as an XML messaging system, providing a framework of services to Member States by means of a reliable and secure exchange of messages. The solution consists of a centrally hosted application offering SafeSeaNet services. The central part of the system is the EIS. The rules and requirements concerning the use of the SafeSeaNet system through XML are described in comprehensive documentation. The SafeSeaNet XML Messaging Reference Guide is an important document covering the message content and format. Copies of this can be accessed from the EMSA website on:

http://www.emsa.eu.int/end806d007.html.

Access to data through the SafeSeaNet EIS is subject to predefined rules that are implemented through a user identification system. All requests trigger a sequential process of acknowledgement and response messages, until the sought after information is received by the requester/user.

2. EMSA’s experience in developing and operating the central system

By the end of 2008, SafeSeaNet will be fully operational with an extensive range of message types. Currently the system approaches 2 million message exchanges per month. This overview shows the SafeSeaNet status of Member States in June 2008.

Member States SafeSeaNet status (June 08)

SafeSeaNet is currently in its Version 1.9 and follows a protocol for Member States to exchange messages with the core (the EIS) using XML. To fully integrate their national information networks within the SafeSeaNet system, the 23 EU maritime Member States (plus Norway and Iceland) must each develop their own XML interface for data handling, particularly for the notifying of information to the EIS. The process of requesting data may be achieved through either of the “XML” or “web” interfaces offered by the system. Access by landlocked EU countries to SafeSeaNet information is achieved only through the web interface (they currently act only as data requesters and not as data providers).

In order to achieve a high level of uniformity in the use of SafeSeaNet EMSA organises coordination between the Member States concerned. For the proper synchronisation of messages by each Member State, it is essential for them to have completed successful tests in a non-live environment provided for testing before going live within SafeSeaNet. As part of the data validation process, EMSA monitors and follows-up implementation, testing and operation of the system to ensure harmonisation of procedures and the successful completion of the tests. It provides helpdesk services, monitors and checks performance and continuity in functionality, follows up implementation of amendments etc.

3. VTS messages proposed at IALA and those used in SafeSeaNet

Development of an inter-VTS exchange protocol by IALA is a useful step forward. The IALA draft standards attempt to establish a baseline for the exchange of information for the interface between adjacent and/or related VTS systems recognising the use of XML as one method for facilitating the exchange.

From reviewing the content of VTS 26/15/02, VTS 26/15/03 and earlier VTS Committee documents it can be concluded that the objectives of SafeSeaNet are broader in terms of data users than the draft IALA guidelines. The SafeSeaNet system is created for the exchange of information between Member State authorities and is not limited only to exchanges between VTS. SafeSeaNet incorporates meeting the need for VTS to VTS exchange as well as for sharing information with other allied services, which accords more closely with the IALA VTS Committee task objectives.

While in principle, harmony between these two standards should be seen as an objective, EMSA and the Member States would all have serious difficulty changing from currently established standards. Every change in redefining the content of SafeSeaNet messages must be agreed with, and then replicated in the Member States national systems.

# 4. Proposed Actions

The IALA VTS Committee is invited to consider and reflect during the development of future standards, that the fullest possible recognition of a comprehensive set of standardised XML messages already exists, being developed and used in the SafeSeaNet system in use throughout the European Union (plus Norway and Iceland).

In addition to any direct concerns regarding the format and content of messages and related to current experience regarding SafeSeaNet, the business logic, workflows, validation of the data quality and security for the exchange of information should also be considered.

While a recognition of how the emergence of XML for data representation and exchange would promote making ship-based information available in a simple and usable format, the absence of any example of the business logic within a future IALA standard would make it difficult to have a full understanding of how messages are exchanged in practice for the benefit of users. This is relevant bearing in mind that the task identified in the agenda item is for guidance on sharing information amongst VTS, with other allied services and the general public. Such matters are effectively dealt with in the SafeSeaNet system.

In support of the issues raised above on the development of standards or guidance, the following questions need also to be clarified:

- What will be the process for requesting and receiving information? Is all information shared or replicated between two or more systems, or is there a manual or ad hoc procedure by which operators can decide what information needs to be transferred or shared with an adjacent VTS?

- Is there a need for a central index? Where is the requested information to be located? How should country A know that information on vessel X is located in country B or C?

- Who coordinates the system including management of messages, the quality of data, the quality of service, management of access rights etc.?