Dear VTS Committee Member,

At VTS 27, an output paper was agreed proposing that council sought clarification regarding “Result Oriented Instructions” (VTS27-output-03).  During VTS 27, the Operations WG also progressed a Working Paper on Navigational Assistance (VTS27-WP4 – now VTS 28/12/01).

Feedback from the IALA Secretariat indicates that Council considered that an approach to IMO would be counter-productive and was sufficiently comfortable with the proposal to direct the VTS Committee to incorporate the spirit of the proposal in IALA documentation.

Since this decision was made by Council, the Chairman of WG1 and the Vice Chairman worked together to further develop working paper VTS27-WP4 out of committee to help move this task on.  We believe that the draft below has incorporated all of the material previously included in WP4 and takes the original draft further forward to maturity.  We, therefore, jointly submit this draft for consideration in the opening plenary as a replacement for WP4 (now VTS28/12/01) and used in it place as the basis for further staffing of this task.

Barry Goldman					Neil Trainor

IHMA     						AMSA

Table of Contents

1	Overview	1

1.1	Scope	2

2	References	3

3	Definitions and Clarifications	4

4	Abbreviations	5

5	Provision of a Navigational Assistance Service	6

5.1	Responding to traffic situations developing in the VTS area	6

5.2	Interaction between a VTS and a vessel	7

5.2.1	Master / VTS Relationship	7

5.2.2	Occasions for Use of Navigational Assistance	7

5.2.2.1	On Request	7

5.2.2.2	Observed	7

5.2.3	Communicating Navigational Assistance	8

5.2.3.1	Result Oriented Messages	8

5.2.3.2	Communication Messages and Message Markers	8

5.2.4	Types of Navigational Assistance	9

5.2.4.1	Information	9

5.2.4.2	Advice	10

5.2.4.3	Instruction	11

5.2.4.4	Warning	12

5.2.4.5	Summary	12

5.3	Operational Procedures	13

5.4	Promulgation of Information and Categorisation of Services	13

5.5	Shore Based Pilotage	13

# Overview

The purpose of vessel traffic services is to improve the safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, worksites and offshore installations from possible adverse effects of maritime traffic.

The principles of vessel traffic services are governed by a hierarchy of regulatory requirements and guidelines.  Key requirements and guidelines include:

- SOLAS Regulation 8-2 “Vessel Traffic Services”

- IMO Resolution A.857(20) Guidelines for Vessel Traffic Services

- Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems

- IMO Resolution A918(22) IMO Standard Marine Communication Phrases

- IALA Vessel Traffic Services Manual

Chapter V “Safety of Navigation” of the SOLAS 1974 Convention, Regulation 8-2 “Vessel Traffic Services”, states, amongst other things, that:

- Vessel traffic services contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.

- Contracting Governments planning and implementing VTS shall, wherever possible, follow the guidelines developed by the Organization.

Implicit in the statement that contracting governments, wherever possible, shall follow guidelines developed by the Organisation, is the need to ensure consistency in the approach taken to the delivery of VTS and to avoid confusion for the mariner trading between various jurisdictions about the delivery of VTS services.

IMO Resolution A.857(20) Guidelines for Vessel Traffic Services define a Vessel Traffic Service (VTS) as a service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

In providing definitions and clarifications with regards to VTS services, IMO Resolution A.857(20) also states that VTS should comprise at least an Information Service and may also include others, such as a Navigational Assistance Service or a Traffic Organisation Service, or both.  Further, it defines a navigational assistance service as a service to assist on-board navigational decision-making and to monitor its effects.

A Navigational Assistance Service is generally initiated in response to traffic situations developing in the VTS area and it is important that information to assist the onboard decision making is provided in a timely manner and is clearly understood by both parties and is not open to misinterpretation to minimise the risk of unexpected and dangerous reactions.

In providing guidance on the implementation of a Navigational Assistance Service by an authorised VTS the IALA VTS Manual (2008) states that:

A Navigational Assistance Service may be provided in addition to an Information Service and/or Traffic Organisation Service. It is a service to assist in the on-board navigational decision-making process and is provided at the request of a vessel, or when deemed necessary by the VTS. A Navigational Assistance Service provides essential and timely navigational information to assist the on-board decision-making process and may inform, advise and/or instruct vessels accordingly.

In providing guidance for the delivery of VTS services IMO Resolution A.857(20) Guidelines for Vessel Traffic Services states that in the provision of information, advice or instruction:

- The communication between a VTS authority and a participating vessel should be conducted in accordance with Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems) and be limited to information essential to achieve the objectives of the VTS, and

- IMO Standard Marine Communication Phrases should be used where practicable (IMO Resolution A918(22) IMO Standard Marine Communication Phrases).

## Scope

The aim of this document is to provide:

- Guidance on the delivery of a Navigational Assistance Service by a VTS to ensure that interaction to assist onboard decision making is consistent between VTS centres, timely and is clearly understood.

- A supplement to IMO Resolution A.857(20) Guidelines for Vessel Traffic Services, Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems) and IMO Resolution A918(22) IMO Standard Marine Communication Phrases), and

- Examples of Navigational Assistance consistent with these guidelines (Annex 2).

# References

| IMO | Resolution A.857(20) Guidelines For Vessel Traffic Services |
| --- | --- |
| IALA | IALA Recommendation V-128 on Operational and Technical Performance Requirements for VTS Equipment |
| IALA | VTS Manual (2008) |
| IMO | Standard Marine Communication Phrases |
| IMO | Guidelines and Criteria for Ship Reporting Systems (resolution MSC.43(64)) |
| IMO | General Principles for Ship Reporting Systems and Ships Reporting Requirements, including Guidelines for Reporting Incidents Involving Dangerous Goods, Harmful Substances and/or Marine Pollutants (resolution A.851(20)) |

# Definitions and Clarifications

The following terms are used in conjunction with vessel traffic services:

| Advice |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to influence others by a Recommendation.”2 Note: The decision whether to follow the ADVICE still stays with the recipient. ADVICE does not necessarily have to be followed but should be considered very carefully. |
| --- | --- | --- |
| Competent Authority |  | The authority made responsible, in whole or in part, by the Government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment. 1 |
| Contracting Government |  |  |
| Information |  | This indicates that the interaction from a VTS Centre “is restricted to observed facts, situations, etc”. 2 Note: This is preferably used for navigational and traffic information, etc. It is for the recipient to decide the appropriate action to take in response to the INFORMATION provided. |
| Instruction |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to influence others by a Regulation.” 2 Note: This means that the sender must have full authority to send such a message. The recipient has to follow this legally binding message unless he/she has contradictory safety reasons which then have to be reported to the sender An instruction may be issued under the authority of a Harbourmaster’s direction. Whilst the responsibility must always remain with the master of a vessel, a decision to ignore an INSTRUCTION may have to be fully justified. |
| Vessel Traffic Services (VTS) |  | A service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.1 VTS should comprise at least an information service and may also include others, such as a navigational assistance service or a traffic organization service, or both, defined as follows1 |
| VTS Authority |  | The authority with responsibility for the management, operation and coordination of the VTS, interaction with participating vessels and the safe and effective provision of the service. 1 |
| VTS Operator |  | An appropriately qualified person performing one or more tasks contributing to the services of the VTS. 1 |
| VTS traffic image |  | The surface picture of vessels and their movements in a VTS area. 1 |
| VTS Centre |  | The centre from which the VTS is operated. 1 |
| Warning |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to inform others about danger.” 2 Note: This means that any recipient of a WARNING should pay immediate attention to the danger mentioned. It is for the recipient to decide the appropriate action to take in response to a WARNING. A WARNING may be supplemented by the inclusion of ADVICE |
| 1 | IMO Resolution A.857(20) Guidelines For Vessel Traffic Services |  |
| 2 | IMO Resolution A.918(22) IMO Standard Marine Communication Phrases |  |

# Abbreviations

| IALA | International Association for Marine Aids to Navigation and Lighthouse |
| --- | --- |
| IMO | International Maritime Organization |
| MSC | Maritime Safety Committee (Standing Committee of IMO) |
| VTS | Vessel Traffic Services |
| SOLAS | Convention on the Safety of Life at Sea |

# Provision of a Navigational Assistance Service

A Navigational Assistance Service is a service that provides essential and timely navigational information to assist in the onboard navigational decision-making process. It may also involve the provision of navigational advice and/or instruction.  It may be provided in addition to an Information Service or Traffic Organisation Service

The efficiency of a VTS will depend on the reliability and continuity of communications and on the ability to provide good and unambiguous information. The quality of accident prevention measures will depend on the system's capability of detecting developing situations and on the ability to give timely information, advice or instruction to assist the on-board decision making.

The services offered to the mariner by a VTS should be promulgated in a manner that clearly communicates details of the VTS, including its capabilities, rules, regulations, requirements and procedures.

Navigational Assistance may be provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary. Such assistance requires positive identification and continuous communication throughout the process.

It is important to recognise that a navigational assistance service:

- Is a service provided to an individual vessel, at its request or when deemed necessary by the VTS, to directly assist the decision making process on board the vessel concerned. This service consists of information relating to a specific vessel and may include warnings, advice and instruction subject to the authorisations of the VTS Staff.

- It is a service that is initiated at the request of a participating vessel or when deemed necessary by the VTS.  As such, it has a defined start and end time of interaction between the participating vessel concerned and the VTS centre.

A Navigational Assistance Service is generally initiated in response to traffic and navigational safety situations developing in the VTS area.  It is important that information to assist the onboard decision making is provided in a timely manner and is clearly understood by both parties and is not open to misinterpretation to minimise the risk of unexpected and dangerous reactions.

## Responding to traffic situations developing in the VTS area

IMO Resolution A857 states that a VTS should at all times be capable of generating a comprehensive overview of the traffic in its service area combined with all traffic influencing factors. The VTS should be able to compile a traffic image, which is the basis for its capability to respond to traffic situations developing in its service area. The traffic image allows the VTS operator to evaluate situations and make decisions accordingly.

To respond to traffic situations developing in the VTS area and to decide upon appropriate actions the acquired data should be processed and evaluated. Conclusions from the evaluation need to be communicated to participating vessels.

Examples of developing situations where Navigational Assistance may be requested or deemed necessary by the VTS include:

- Potential collision between vessels

- Potential collision with hazard

- Risk of grounding

- Vessel off course

- Critical overtaking

- Assistance to anchor area

## Interaction between a VTS and a vessel

### Master / VTS Relationship

Vessels navigating in an area where vessel traffic services are provided should make use of these services. Depending upon governing rules and regulations, participation in a VTS may be either voluntary or mandatory.

Where a VTS provides navigational assistance, irrespective of whether the service is initiated by the VTS or the participating vessel, care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot (IMO Resolution A.857(20) Guidelines For Vessel Traffic Services).

Decisions concerning the actual navigation and the manoeuvring of the vessel remain with the master. Neither a VTS sailing plan, nor requested or agreed changes to the sailing plan can supersede the decisions of the master concerning the actual navigation and manoeuvring of the vessel.

### Occasions for Use of Navigational Assistance

The IALA VTS Manual 2008 identifies that Navigational Assistance is a service that is initiated at the request of a participating vessel or when deemed necessary by the VTS.  As such, it has a defined start and end time of interaction between the participating vessel concerned and the VTS centre.

#### On Request

Navigational Assistance may be provided on request by a vessel as a back-up either due to equipment failure or due to unfamiliarity.  It may be pre-planned or requested in an emergency.  Individual circumstances will dictate the degree of preparation that can be undertaken prior to commencing a Navigational Assistance Service on Request but, wherever possible preparations should include an assessment of the capability of the vessel to undertake the passage safely and/or the risks involved in declining assistance if the request is sought after the vessel has entered restricted waters.

The provision of Navigational Assistance does not absolve the master from his responsibility for the safety of his own ship; in particular the Master should be made aware of the limitations on VTS radar and the specific responsibilities of the Master for collision avoidance.  If possible and if time permits, checks should normally be made prior to commencement of the provision of Navigational Assistance to assess the capability of the vessel to respond to the guidance given.  An example checklist is at Annex 1, which should be modified as required for local requirements.

#### Observed

Navigational Assistance may be provided when the VTS observes from a vessel’s actions that intervention is necessary.  Under such circumstances, it is likely that the priority will be on the provision of the necessary navigational guidance and advice under the Navigational Assistance before attempting to formally define the time that Navigational Assistance has started.  However, once the immediate problem has been resolved, the continuation or completion of Navigational Assistance should be subsequently clarified and the use of the check list considered.

### Communicating Navigational Assistance

In providing navigational assistance it is important that the interaction from a VTS centre  to assist the onboard decision making is conducted in a timely manner, is clearly understood by both parties and is not open to misinterpretation. In providing guidance for the delivery of VTS services IMO Resolution A.857(20) Guidelines for Vessel Traffic Services states that in the provision of information, advice or instruction:

- The communication between a VTS authority and a participating vessel should be conducted in accordance with the Guidelines and Criteria for Ship Reporting systems and should be limited to information essential to achieve the objectives of the VTS (Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems), and

- IMO Standard Marine Communication Phrases should be used where practicable (IMO Resolution A918(22) IMO Standard Marine Communication Phrases).

- A Navigational Assistance Service is a service that is initiated at the request of a participating vessel or when deemed necessary by the VTS.  Messages relating to Navigational Assistance must always be addressed by name to the vessel participating in Navigational Assistance so that there is no doubt to whom the content of the message is directed.

- Consideration should be given regarding the communications frequency on which the Navigational assistance Service should be provided.  The decision will depend on individual and local circumstances.  However, an assessment should be made of the relative benefits of conducting the assistance on a discrete frequency so that interference from other users can be avoided, or the use of  a common working frequency such that other users are aware of the likely actions of the vessel participating in the Navigational Assistance Service.  Other options may be available if the participating vessel is able to monitor two of more frequencies.

#### Result Oriented Messages

A fundamental principle of VTS communications is that, when authorised to provide Navigational Assistance to vessels, any instruction communicated should be result-oriented only; leaving the details of execution, such as course to be steered or engine manoeuvres to be executed, to the person on board accountable for navigational decision making at that time.

The interpretation of “Result Oriented” will depend on the situation and context. Phrases that are used for vessel conning, such as, “Stop Engine”, “Hard to Starboard” or “Steer Course “XXX”” should not be used.  If it is considered appropriate to issue guidance on courses to be steered or speed, then it is important that it is made clear that such guidance is given as a recommendation and that the decision whether to follow the ADVICE still stays with the recipient.  Use of the ADVICE message marker should be considered to ensure that there is no doubt regarding where the final responsibility lies for acting on the guidance given.

#### Communication Messages and Message Markers

To facilitate shore-to-ship communications in a VTS environment, one of the following eight message markers should be used to increase the probability of the purpose of the message being properly understood. It is at the discretion of the shore personnel or the ship's officer whether to use one of the message markers and, if so, which marker is applicable to the situation. However, since the provision of Navigational Assistance is likely to have arisen out of some form of shortcoming, failure or emergency, the contents of all messages directed to a vessel should be clear and it is recommended that Message Markers from the IMO Standard Marine Communication Phrases are always used in the delivery of a Navigational Assistance Service.  If used, the message marker is to precede the message or the corresponding part of the message.

There are eight types of Communication Messages that are frequently used in VTS (These are defined in SMCP, see 1709) each with its own discrete message marker. These are:

| Information | Warning | Advice | Instruction |
| --- | --- | --- | --- |
| Question | Answer | Request | Intention |

For more information see the IMO Resolution A918(22) IMO Standard Marine Communication Phrases.

### Types of Navigational Assistance

In providing essential and timely navigational information to assist the onboard decision-making process a VTS may provide:

- Information,

- Warning,

- Advice, and/or

- Instruction

IMO Resolution A.857(20) Guidelines For Vessel Traffic Services states that in the interaction between a participating vessel and a VTS centre:

- A distinction should be made between the provision of navigational information, being a relay of information extracted from the VTS sensors and the traffic image, and the provision of navigational advice, where a professional opinion is included, and

- In any message directed to a vessel it should be clear whether the message contains information, advice, warning or instruction and that IMO Standard Communication Phrases should be used where practicable.

Further guidance on the types of interaction available to a VTS Centre when responding to traffic situations developing in the VTS area and interacting with the traffic to ensure essential and timely information is communicated to assist the on-board decision-making process is provided in Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems and  IMO Resolution A918(22) Standard Marine Communication Phrases.

#### Information

IMO Resolution A918(22) IMO Standard Marine Communication Phrases defines Information as communication whereby the message is restricted to observed facts, situations, etc. and is preferably used for navigational and traffic information, etc..   As such, it is a relay of information extracted from the VTS sensors and the traffic image where no professional opinion by the VTSO is included.

Implicit in this definition is that the consequences of using the “INFORMATION” will be up to the recipient.

Examples of the provision of an Information Message during the delivery of Navigation Assistance to a participating vessel include:

- Course and speed made good by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards; and

- Positions, identities, intentions and any restrictions of surrounding traffic.

For example:

| Example 1 | “INFORMATION, By VTS radar, vessel “No Name” will overtake you in the vicinity of Buoy “XXXX”. |
| --- | --- |
| Example 2 | “INFORMATION, Next high water at Port “YY” predicted to be “XX:XX” at a height of “ZZ” metres. |
| Example 3 | “INFORMATION”, VTS radar indicates you are heading towards shallow water”, distance X.X . |
| Example 4 | “INFORMATION”, VTS radar indicates you are heading towards shallow water”, distance X.X . “INFORMATION”, Safe water is bearing xxx degrees from you. |

#### Advice

IMO Resolution A918(22) IMO Standard Marine Communication Phrases defines advice as communication whereby the message implies the intention of the sender to influence the recipient by a recommendation.

Implicit in this definition is:

- A professional opinion is included in the message as a means to influence the recipient, and

- The recipient should pay immediate attention to the advice mentioned and the consequences of using the information provided will be up to the recipient.  Advice does not necessarily have to be followed but should be considered very carefully by the recipient. The decision whether to follow the advice remains with the recipient.

The provision of advice in response to a developing situation may also include or require:

- An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- An assessment of the environmental conditions;

- An assessment of the implications of the cargo carried;

Examples of the provision of Advice during the delivery of Navigation Assistance, to a participating vessel include:

| Example 1. | “ADVICE, By my display you are leaving the channel. Recommend you transit within the channel”. |
| --- | --- |
| Example 3 | ADVICE. Recommend you: ~ Make good a course of ….. degrees. ~ Steer a course of … degrees. |

Note: the user of this phrase should be fully aware of the implications of words such as "track", "heading" and "course made good".

Track - The path followed, or to be followed, between one position and another

Heading - The horizontal direction of the vessel's bows at a given moment measured in degrees clockwise from north

Course made good - That course which a vessel makes good over ground, after allowing for the effect of currents, tidal streams, and leeway caused by wind and sea

#### Instruction

IMO Resolution A918(22) IMO Standard Marine Communication Phrases defines Instruction as communication whereby the message implies the intention of the sender to influence the recipient by a Regulation.

Implicit in this definition is:

- The message is a direction given by the VTS centre under the provisions of a statutory regulation.

- The sender must have delegated authority to communicate such a message.

- The recipient has a legal obligation to comply with this message unless contradictory safety reasons exist, which then have to be reported immediately to the sender.

Generally masters of vessels will respond promptly and carry out instructions given by a VTS. However, it should be recognised that there may be occasions when an instruction by a VTS is disregarded because the master has additional information not available to the VTS Centre and he decides on another course of action. For example, a vessel that is not being tracked by the VTS may be a contributing factor to the navigational situation.

Examples of the provision of Instruction during the delivery of Navigation Assistance, to a participating vessel include:

| Example 1 | "INSTRUCTION. Do not cross the fairway." |
| --- | --- |
| Example 2 | “INFORMATION. Visibility is less than 200 meters. “INSTRUCTION. Do not enter the Fairway” |

Note - When the VTS is authorized to issue instructions to vessels, these instructions should be result-oriented only, leaving the details of execution to the master or pilot on board the vessel. Care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot.  Thus, whilst it is acceptable to issue ADVICE on heading and speed, it would not be appropriate to issue heading and speed requirements as an INSTRUCTION.

#### Warning

The provision of Information during the delivery of Navigation Assistance may also include Warnings.  IMO Resolution A918(22) IMO Standard Marine Communication Phrases defines Warning as communication whereby the message implies the intention of the sender to inform others about danger.  It may be used to convey potentially dangerous situations or observed developing situations.

As such, it is a relay of information extracted from the VTS sensors and the traffic image and, in the professional opinion of the VTSO, the message should be communicated to inform a vessel about potential danger.

The contents of a Warning Message should be immediately assessed onboard the vessel in conjunction with any additional information which may not be available to the VTS centre and corrective action taken where necessary.  A Warning Message may be supplemented by further messages such as ADVICE or INSTRUCTION

Implicit in this definition is that the recipient should pay immediate attention to the danger mentioned.  The provision of further messages such as ADVICE or INSTRUCTION may assist the recipient in assessing the consequences of using the information provided.  Examples of the provision of a Warning Message during the delivery of Navigation Assistance to a participating vessel include:

| Example 1 | "WARNING. Obstruction in the fairway. Submerged container …. degrees, distance XX meters from ?? Buoy.” |
| --- | --- |
| Example 2 | “WARNING”. VTS radar indicates you are heading towards shallow water, distance X.X .” |
| Example 3 | “WARNING. VTS radar indicates you are heading towards shallow water, distance X.X .” “ADVICE. Recommend you steer a new course of …… degrees.” |
| Example 4 | “WARNING”. VTS radar indicates you are now very close to shallow water” “ADVICE, Recommend you stop your ship immediately” |
| Example 5 | “WARNING, By VTS radar you are dragging your anchor. “QUESTION” What are your intentions”. |
| Example 6 | “WARNING. The large Container Vessel ‘LCV Europa’, is experiencing difficulty swinging in the fairway off your berth. “INSTRUCTION. Do not proceed.” |

#### Summary

The table below summarises the message markers that should be adopted with the respective category of service provided for the provision of essential and timely navigational information to assist the onboard decision-making process and when it is assessed that SMCP should be used.

| Interaction | VTS Service Provided |  |  |
| --- | --- | --- | --- |
|  | Information Service | Traffic Organisation Service | Navigational Assistance Service |
| Information | Y | Y | Y |
| Warning | Y | Y | Y |
| Advice |  | Y | Y |
| Instruction |  | Y1 | Y1 |

1Note - When the VTS is authorized to issue instructions to vessels, these instructions should be result-oriented only, leaving the details of execution to the master or pilot on board the vessel. Care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot.  Thus, whilst it is acceptable to issue ADVICE on heading and speed, it would not be appropriate to issue heading and speed requirements as an INSTRUCTION.

## Operational Procedures

Clear operational procedures should be in place for the provision of Navigational Assistance when requested by a vessel, or when observed and intervention is deemed necessary by the VTS. The authorisation of VTS personnel to provide this service should also be identified. VTS Authorities should give careful consideration to staffing levels, their qualifications and equipment capability when implementing a Navigational Assistance Service.

## Promulgation of Information and Categorisation of Services

The services offered to the mariner by a VTS should be promulgated to vessels in the appropriate internationally recognised marine publications. This should include details of the VTS, its capabilities, rules, regulations, requirements and procedures. The information promulgated should be verified and up-dated at least at annual intervals.

The IMO has set out the requirement for the publication of VTS information as follows:

The VTS Authority should, in a timely manner, provide mariners with full details of the requirements to be met and the procedures to be followed in the VTS area. This information should include the categories of vessels required or expected to participate; radio frequencies to be used for reporting; areas of applicability; the times and geographical positions for submitting reports; the format and content of the required reports; the VTS authority responsible for the operation of the service; any information, advice or instructions to be provided to participating ships; and the types and level of services available.

## Shore Based Pilotage

Shore Based Pilotage is a term that has been coined by some authorities for the direction of vessel traffic from ashore.  It is authorised on a national basis as a pilotage function and the SMCP references relating to Shore Based Pilotage should be considered in this regard.  Shore Based Pilotage is not a VTS function and is not considered further in this guideline.

Annex 1

EXAMPLE1

VTS CHECKLIST FOR THE PROVISION OF NAVIGATIONAL ASSISTANCE

VTS:- …………………

VESSEL:-……………………………	MASTER’S NAME:-…………………

Review:

- Vessel’s Machinery State

- Vessel’s Navigational equipment State

- Up-to-Date charts

- Master/Officer knowledge of English/Local language

- Master’s knowledge of Local Area

- Environmental Conditions (Wind, Day/Night, Visibility, Tidal Height, Tidal Stream)

- Traffic Levels

- Cargo

- VTS Operator Competence/Authorisation

- VTS Equipment performance, serviceability and back-up (particularly key elements of communications, radar and AIS)

Assess and Decide:

- Capability of vessel to continue passage under VTS NAS.

- The risk of not providing NAS if the vessel is already in restricted waters.

- Need for amendment to passage plan (such as a temporary anchorage).

- Alternative options such as provision of a pilot.

- Communications channel for provision of NAS.

Agree:-

  - Master’s acceptance to continue passage under Navigational Assistance.

  - Master’s understanding that NAS is advisory and does not in any way absolve him from his responsibility for the safety of his vessel.

  - Responsibilities for Collision Avoidance.

  - Commencement of NAS

  - Completion of NAS

1 This checklist is an example, which should be modified as required for local requirements as required through risk assessment.

A Warning message is a particular type of  SMCP communication message that may be appropriate for use in drawing attention to the significance of a message providing “information” during NAS as described in the IALA VTS Manual Art 0502