Guideline

G1117

VHF Data Exchange System (VDES) Overview

Preliminary Draft Revision of Edition 2.0

March 3rd, 2022, JWG IALA eNAV WG3/IEC TC80/WG15

Revisions to this IALA document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| December 2016 | 1st issue | Council 63 |
| December 2017 | Edition 2 Updated to reflect technical developments in VDES and to reflect transmission aspects (addressed / broadcast) | Council 65 |

Revision by  working group

| Date | Details |  |
| --- | --- | --- |
| 28 February 2022- 3 March 2022 | JWG IALA/IEC Intersessional workshop, multiple authors; the work of all contributions at the workshop is included in this version (also from Pieter Winter, but as edits of Stefan Pielmeier due to technical reasons, see ANNEX). | After eNAV29 |

List of Tables

Table 1	Maritime Service, IMO update 1 - e-Navigation Strategic Implementation Plan	7

Table 2	Considerations for VDES Implementation	14

Table 3	Comparison of Digital Communications Technologies	16

Table 4	Potential VDES Uses cross-referenced to IMO SIP MS update 1	17

Table 5	Frequencies allocated to VDES	30

Table 6	VDE link layer messages	34

List of Figures

Figure 1	Implementation of VDES	5

Figure 2	VDES functions and frequency use – full system	7

Figure 3	Concept for VDES	9

Figure 4	Concept for VDES with Martime Connectivity Platform [WorkInProgress]	10

Figure 5	VDES implementation decision matrix	13

Figure 6	Increasing data bandwidth in VDES	30

Figure 7	VDES ship station logical description	32

Figure 8	VDES shore station logical description	32

Figure 9	VDES into the Common Shore Side Architecture	33

Figure 10	Ship control station	34

# INTRODUCTION

## Purpose of the document

This Guideline provides an introduction to the VHF Data Exchange System (VDES) at an overview level. It does not include technical details.

This document is intended to assist in the understanding, development and promotion of VDES.

## Background

<<Add revision of appendix 18 in 2019>>

AIS is well recognized and accepted as an important tool for safety of navigation and is a carriage requirement for SOLAS vessels (Class-A). With increasing demand for maritime VHF data communications, AIS has become heavily used for maritime safety, maritime situational awareness and port security.  As a result, overloading of AIS 1 and AIS 2 created a need for additional AIS channels. Using the VHF marine band (International Radio Regulations Appendix 18) AIS can broadcast data to vessels in the vicinity of the AIS unit. AIS can also transmit an addressed message.

International Telecommunications Union (ITU) has recognised the efficiency and the necessity for digital communications, has produced technical standards and has revised the VHF marine band (Radio Regulations Appendix 18) to designate channels for data transmission.  It is recognized that both analogue voice communications and digital communications will share the band. The VDES, as envisioned by IALA and presented to ITU, addresses the identified need to protect AIS along with essential digital communications contributions for e-Navigation and GMDSS Modernization.

The VHF marine band (Radio Regulations Appendix 18) was initially used for transmission of voice communications on 25 kHz channels. The ITU introduced the first marine data transmission system, DSC (Digital Selective Calling) to help ensure that calling and distress communications attempts were successful. VHF DSC transmits data at 1.2 kbps, slow by modern data standards, but very robust.  At the request of the IMO to improve safety of navigation, ITU introduced another VHF data transmission system, AIS, which provides navigation and identification data for ships, shore stations, aids to navigation and search and rescue devices at 9.6 kbps.

ITU introduced a standard, with options for 25 kHz, 50 kHz and 100 kHz channels at data rates up to 307.2 kbps in order to improve spectrum efficiency in 2012.  Both voice and data communications coexist in the VHF marine band.  The developments in maritime radio technology, including the introduction of software defined radios (SDR) coupled with enhanced capabilities for digital data exchange over existing VHF marine band spectrum resulted in the development of the VHF Data Exchange System (VDES).  VDES builds on the experience gained through the development of AIS, and also provides the capability to transmit to a specific vessel (addressed); to all units in the vicinity (broadcast); to a group of vessels (addressed); or to a fleet of vessels (addressed).

Consequential to WRC-15, the ITU standard for VDES, Recommendation ITU-R M.2092-0, was approved.  The remaining approval of the satellite component for the VDE channels was approval at WRC-19.

During the WRC-19 it was also approved that for the ARMD Group-B a new channel was assigned within the VHF band. This decision does not affect the VDES.

2020

2021

2018

2019

2016

2017

2022

2023

AIS + VDE Terrestrial Initial Operational Capability

VDES Terrestrial Initial Operational Capability

VDES Full Operational Capability

AIS

AIS 1
AIS 2

VDE-SAT uplink/downlink

VDE-TER

VDE-TER + ASM (incl. uplink)

VDE-TER; VDE-SAT + ASM

VDE-TER

ASM 1
ASM 2

2020

2021

2018

2019

2016

2017

2022

2023

AIS + VDE Terrestrial Initial Operational Capability

VDES Terrestrial Initial Operational Capability

VDES Full Operational Capability

AIS

AIS 1
AIS 2

VDE-SAT uplink/downlink

VDE-TER

VDE-TER + ASM (incl. uplink)

VDE-TER; VDE-SAT + ASM

VDE-TER

ASM 1
ASM 2

Implementation of VDES

The introduction of VDES is expected to happen through four operational phases:

(2016) AIS exists as defined by ITU.R M.1371-5 on the AIS frequencies, and Coastal Stations use the ASM and VDE frequencies for Voice VHF.

(2017-2018) Post WRC-15 - AIS+ASM: Regionally, where there is an urgent need for offloading the AIS VDL from significant ASM traffic, it is recommended to allow the introduction of 4-channel AIS + ASM devices. These devices may receive and transmit ASM on the ASM1 and ASM2 frequencies, but shall discontinue their transmit capability, using the existing GMSK modulation after January 1st 2019 unless a software upgrade enables them to participate in the modulation and access scheme agreed for the ASM frequencies. Note that the ASM frequencies will need to be shared with the VHF voice service from Coast Stations in many areas during this time frame.

(2019) at the first of January 2019 the VDES VHF channels should only be used digital according to ITU. This led to operational problems and IMO postponed (MSC.1/Circ.1460/Rev.2 ) the use until the 1st of January 2024. ITU decided at the WRC-19 regarding VDE-SAT that frequencies were added to the VDES.

(2019-2020) Post WRC-19 operational capability established. Note that both the ASM and VDE frequencies may still need to be shared with the voice VHF service in many areas.

(2021+) When a satellite service is developed, full operational capability of the VDES including the Satellite frequencies can be achieved.

(2024) All the VDES VHF channels shall only be used for the VDES (MSC.1/Circ.1460/Rev.2)

## Overview

The VHF Data Exchange System (VDES) is seen as an effective and efficient use of radio spectrum, building on the capabilities of AIS and addressing the increasing requirements for data through the system.  New techniques providing higher data rates than those used for AIS is a core element of VDES. Furthermore, VDES network protocol is optimized for data communication so that each VDES message is transmitted with a high confidence of reception. VDES increases the capability for digital data exchange in a manner similar to AIS, which includes provision of data to vessels in a geographic area (broadcast), to a specific vessel or a group of vessels in a geographic area (addressed) or to a fleet of vessels (addressed).

In this document, when communications from ship to shore are referenced, this includes ship to satellite to shore and shore to satellite to ship.  It is noted that, following WRC-19, the full satellite capability of VDES is available from the ITU decision. The operational use will be from the 1st of January 2024 ((MSC.1/Circ.1460/Rev.2).

# GENERAL DESCRIPTION

The VDES should improve the safety of life at sea, the safety and efficiency of navigation, and the protection of marine environment and enhance maritime safety and security.  These goals will be achieved through efficient and effective use of maritime radiocommunications, incorporating the following functional requirements:

- As a means of AIS.

As a means of radiocommunications equipment through exchange of digital data between ship and ship, ship and shore including satellite via AIS, Application Specific Messages (ASM) and VHF Data Exchange (VDE).

As a means of applications external to the VDES equipment itself.  These applications use AIS, ASM or VDE separately or combined.

## System concept

The VDES concept was originally proposed to address emerging indications of overload of the VHF Data Link (VDL) of AIS and simultaneously enable a wider seamless data exchange for e-navigation, potentially supporting the modernization of GMDSS.  In addition, VDES could support the increasing communications requirements identified through the development of e-Navigation, as documented in the e-Navigation Strategic Implementation Plan (SIP) IMO MSC.1/Circ.1595.

The purpose of e-Navigation is to enhance berth-to-berth navigation and related services for safety and security at sea and protection of the marine environment.  e-Navigation seeks to enhance maritime safety through simplification and harmonization of information.  In addition, e-Navigation seeks to facilitate and increase efficiency of maritime trade and transport by improved information exchange.

The VDES system concept recognises the parallel work being carried out related to Maritime Service in the context of e-Navigation former known as Maritime Service Portifolio (MSP).  Where applicable, these MSP are referenced in this user requirements document.

Table 1 identifies the Maritime Service in the context of e-Navigation  as defined by IMO e-navigation strategic implementation plan (SIP) and its update E-Navigation Strategy Implementation Plan - Update 1 (IMO MSC.1/Circ.1595).

| Service No | Service | Domain coordinating body | Identified responsible service provider |
| --- | --- | --- | --- |
| MS 1 | VTS Information Service (INS); | IALA | VTS Authority |
| MS 2 | Navigation Assistance Service (NAS) | IALA | VTS Authority |
| MS 3 | Traffic Organization Service (TOS) | IALA | VTS Authority |
| MS 4 | Local Port Service (LPS) | IHMA | Local Port/Harbour Authority |
| MS 5 | Maritime Safety Information (MSI) service | IHO | National Competent Authority |
| MS 6 | Pilotage service | IMPA | Pilotage Authority/Pilot Organization |
| MS 7 | Tugs service | TBD | Tug Authority |
| MS 8 | Vessel Shore Reporting | TBD | National Competent Authority and appointed service providers |
| MS 9 | Telemedical Maritime Assistance Service (TMAS) | TBD | National Health Organization/dedicated health Organization |
| MS 10 | Maritime Assistance Service (MAS) | TBD | Coastal/Port Authority/Organization |
| MS 11 | Nautical Chart Service | IHO | National Hydrographic Authority/ Organization |
| MS 12 | Nautical Publications Service | IHO | National Hydrographic Authority/ Organization |
| MS 13 | Ice Navigation Service | WMO | National Competent Authority/Organization |
| MS 14 | Meteorological Information Service | WMO | National Meteorological Authority/Public Institutions |
| MS 15 | Real-time hydrographic and environmental information Services | IHO | National Hydrographic and Meteorological Authorities |
| MS 16 | Search and Rescue Service | TBD | SAR Authorities |

Maritime Service, IMO update 1 - e-Navigation Strategic Implementation Plan

The system concept, including VDES functions and frequency usage are illustrated pictorially in Figure 2 (full system).

VDES functions and frequency use – full system

## Concept of operations

The key concept of operation of the VDES includes:

- The VDES provides a capability of data exchange between ships and shore users by terrestrial or satellite link.

- The VDES can operate without a shore infrastructure.

- Data exchange from the ship may occur automatically or manually.

- Data exchange uses the designated VHF channel(s).

- Transmission and reception of the data occurs with the minimum involvement of ship’s personnel.

- The VDES includes existing AIS applications.

- The VDES includes existing ASM.

- VDES additional capabilities include support of the VHF Data Exchange (VDE).

- The VDES related applications should support language independent communications (e.g. through the use of a digital data dictionaries).

- The VDES implements data integrity monitoring at the VDES link level (e.g. check sum).

- The VDES related applications address cyber security (e.g. authentication, key management and, if required, encryption).

- The VDES has a high level of availability.

- The VDES supports machine-to-machine communications (for example, interfaces with external equipment providing applications related to VDES).

- The VDES related applications enable clear comprehension of the information sent / received through the VDES.

The concept of operations is identified in Figure 3.

Concept for VDES

### Architecture using Maritime Connectivity Platform

VDES can be used as data transport medium in the maritime domain to support many applications.

For applications that require the above listed key features of

  - language independent and machine-to-machine communications,

  - cyber security, and

  - international service discovery,

the concept of the maritime connectivity platform [editor: add reference to https://maritimeconnectivity.net/ ] contributes with major key building blocks

  - Maritime Identity Register (MIR),

  - Maritime Service Registry (MSR), and

  - Maritime Messaging Service (MMS).

The Maritime Identity Register MIR is a decentral register of maritime digital communication participants and provides trust between individual participating parties in the maritime digital communication domain. Trust is provided by issuing unique digital certificates to users based on their individual maritime resource name (MRN) [editor: add reference to IALA MRN specification]. Participants can use the MIR to verify other participants’ signatures for authentication, to fetch public certificates needed for encryption and update revocation lists to invalidate cached certificates that were reported outdated or stolen. Participants listed in the MIR are secured by a vetting procedure, ensuring their authenticity. Each participant in the maritime digital communication domain is using a list of trusted MIR servers. IALA will provide a list of trusted MIR servers. The content of the MIRs is cacheable on board to allow ad hoc ship-ship communications without requiring direct connectivity to a MIR in real time. A broadcast authentication protocol could be the TESLA (Timed Efficient Stream Loss-tolerant Authentication) protocol that uses a combination of asymmetric (public key-based) and symmetric (secret key) cryptography to minimise protocol overhead to meet the VDE-TER capabilities.

The Maritime Service Registry MSR is a decentral register listing vetted and quality checked services with their browsable and searchable key parameters such as:

- Service MRN,

- Service Name,

- Service Topic,

- Service geographical area,

- Digital format used.

The Maritime Messaging Service MMS is a concept comprised of components that allow exchange of digital binary “messages”, i.e. arbitrary digital data, organized in a predefined known way to be used by digital applications. The MMS provides a non-synchronous data exchange, that can be realized over changing, intermittent and heterogenous digital connectivity. The MMS supports both secure IP based transport through internet connectivity and through VDE-TER and VDE-SAT connections, and thus provides a solution to fulfil the requirements to securely transport trusted services and private communication between all maritime participants who at least have VDES or IP connectivity sometimes.

The concept of operations is identified in [editor, please add reference to Figure 4].

Concept for VDES with Martime Connectivity Platform [WorkInProgress]

VDES

IP connection

(VSAT, Iridium, 3G,

cable, etc.)

ECDIS Ed

.

4

Legacy

Systems

Backbridge

-

/

Planning Station

Active Route

Route negotiation

/

Text Chat

Standardized route

exchange format

(

IEC

61174

Ed

.

4

)

Communication Router with VTS

(

select AIS

/

VDES

/

VSAT

)

Route Optimization

/

Flow Management

Handling VDES messages

(

e

.

g

.

MSP

,

etc

)

App

App

App

App

App

App

App

App

App

App

VTS Shore Center

AppServ

App

Other Shore Service

Instances

App

AppServ

LAN

MMS Agent

VDE

AIS

ASM

IEC 61162-450

VDE-SAT

network

Internet

VDE-TER

network

MMS

Agent

MMS

Agent

MMS

Routers

IEC 61162-1

[More to come WIP, see actions from JWG]

## General

The VDES operates according to ITU-R M.2092, which includes the following operational characteristics:

- The system gives its highest priority to the automatic identification system (AIS) position reporting and safety related information.

The system installation is capable of receiving and processing the digital messages and interrogating calls.

The system installation operates continuously while under way, moored or at anchor.

The system, for the terrestrial links, uses the appropriate time-division multiple access (TDMA) techniques, access schemes and data transmission methods in a synchronized manner.

The system is capable of various modes of operation, including the autonomous, assigned and polled modes.

The system prioritizes applications and adapts parameters of the transmission (robustness or capacity) while minimizing system complexity.

## System Overview

The VDES includes:

- Antenna(s), capable of transmitting and receiving data through terrestrial and satellite link.

An AIS as set out in resolution MSC.74(69) ANNEX3.

A multi-function data communication and timing process that is interoperable with AIS, ASM and VDE.

A multi-function transmitter, capable of operating on the designated AIS, ASM and VDE frequencies.

Multi-function receivers, capable of operating on the designated AIS, ASM and VDE frequencies.

A means to automatically input data from other sources.

A means to automatically output data to other devices.

A means of ensuring the integrity of the data.

A means to automatically or manually update the device software as needed.

Functionality of a built-in test equipment (BITE).

## Assumptions and dependencies

The applications related to the VDES address the following assumptions and dependencies:

- VDES operates within the existing AIS environment.

VDES respects and supports requirements for GMDSS communications, including SAR, urgency, and safety related messages.

VDES applications are uniquely identified.

The VDES related applications operate in a manner that ensures there is no unnecessary repetition of messaging.

The VDES could be used for PNT

In principle a AIS system could be replaced by a VDES system.

## VDES SERVICES

Deciding to implement a VDES service follows the decision matrix provided in Figure 4:

VDES implementation decision matrix

Is there a need to use VDES?

Continue to use AIS

Keep under review

Define services to be provided

Plan integration of VDES with AIS

Redefine services to be provided

Existing facilities and equipment

New facilities and equipment

Human resources required

Are sufficient funds available?

Are more funds available?

Implement VDES services

No

No

Yes

Yes

Yes

No

Is there a need to use VDES?

Continue to use AIS

Keep under review

Define services to be provided

Plan integration of VDES with AIS

Redefine services to be provided

Existing facilities and equipment

New facilities and equipment

Human resources required

Are sufficient funds available?

Are more funds available?

Implement VDES services

No

No

Yes

Yes

Yes

No

VDES shore station operations should consider:

AIS system load;

services to be provided;

existing equipment (for existing sites);

antenna requirements, including interaction with other services/systems;

power requirements;

installation costs (which may include mobilisation, tower access);

operational costs

coordination with other shore stations

Provision of VDES shore stations may be managed through:

a regular scheduled lifecycle process for existing AIS shore stations;

through an upgrade of existing AIS shore stations; or

through the implementation of VDES at a new location where there is no existing AIS shore station.

Table 2 provides further information on considerations for VDES implementation.

| Consideration | Detail | AIS recommended | VDES recommended |
| --- | --- | --- | --- |
| AIS system load | AIS system load (see measurement technique in note 1) | <50% load | >50% load |
| Application Specific Messages | Application Specific Messages (ASM) are delayed beyond that acceptable to the users (see note 2) | Service Latency expected | Service Latency not expected |
| Advanced data transfer services | If the Advanced system data transfer features are required as per the Maritime Service Portfolios | Not required | Are required |

Considerations for VDES Implementation

Note 1: System load is determined by measuring the number of vessels in the coverage area of an AIS base station (a single slot map) and calculating the sum of the slots consumed in the coverage area based on the expected reporting rates of the vessels in the coverage area (Class A, Class B, AtoN and AIS base station) and the number of Addressed Binary Messages (ABM) and Broadcast Binary Messages (BBM) of all types.

Note 2: Service latency is defined as the delivery of the total service from the initiation of the delivery of the service component to the completion of the delivery of that service component e.g. a single maritime chart update.

The services offered by a VDES system will allow for priority with essential services / safety related services having the highest priority and non-essential / commercial service having the lowest priority.

The transfer of data using VDES should consider that the available VDES data transfer capacity is shared by all users within the coverage range of a VDES base station.

A technical overview of VDES is provided in Annex A.

## digital communication systems

<<Other means could be considered but take into account specific characteristics of these communication system

- Near real time small messages

- No streaming or large files

- Doesn’t need shore infrastructure

- No need for subscription at provider

- Support for PNT consistency system near shore (R-mode)

>>

Digital communications permeate our daily lives – both for work and for recreation.  The introduction of ‘any-time, any-where’ access to information through the internet; text based or image based communications; geospatial locating and more is driving demand for faster, more robust, and more integrated, communication solutions.

There are a number of existing and developing communications technologies.  These include:

NAVDAT: this system is a development on the current NAVTEX system. The service will support the same major functions as NAVTEX, namely navigational warnings, weather forecasts and emergency information for shipping, but will provide a much greater capacity and data rate.

VHF Data Exchange System: VDES, developed to meet the increasing need for data communication between maritime users and due to the significant rise in VHF data link load with the increasing use of AIS. Provides faster data transfer rates with greater integrity than current VHF data link systems.

Digital Selective Calling: DSC transmits packets of data over existing maritime radio spectrum, on VHF, HF and MF. The system uses maritime mobile service identities and enables direct transmission or group / area transmissions of basic data.  DSC is included in the Global Maritime Distress and Safety System (GMDSS) as a distress and calling mechanism.

Digital VHF Digital VHF is the evolution of analogue based mobile radio systems currently used by mariners for voice communication, transmission / reception of distress and safety information, and reception of urgent marine information broadcasts. As well as digitally encoding voice transmissions digital VHF will enable the exchange of digital data messages. This may be a longer-term development.

Digital HF: Digital HF is the evolution of analogue based mobile radio systems currently used by mariners for voice communication, transmission/reception of distress and safety information, and reception of urgent marine information broadcasts. As well as digitally encoding voice transmissions digital HF will enable the exchange of digital data messages.  Digital HF, including standards for digital HF.

Wi-Fi: provides local area wireless data transfer using the 2.4 GHz to 5 GHz radio wave band. However, the coverage of this system is limited to within a port or harbour environment.

IMT: International Mobile Telecommunications including:

  - 4G: a mobile telecommunications standard supporting mobile internet broadband, succeeding 3G. Provides mobile broadband with data rates of 100s of Mbps for mobile users. Systems falling under the 4G standard include WiMAX and LTE.

  - 5G: planned evolution of 4G, with data rates expected to be 1 Gbps and an intended delivery year of 2020.

Satellite communication systems and services including:

Inmarsat Global Express – GEO satellite constellation.  The latest set of services to be offered by Inmarsat including shared channel IP packet-switched internet broadband service with fast data rates provided by satellites in the Ka band with global coverage.

Inmarsat C – GEO satellite constellation.  Existing short burst data, store and forward system providing low data rates for small message size transfers and also supporting the Global Maritime Distress and Safety Service (GMDSS).

Iridium – LEO satellite constellation.  Existing low-earth orbiting communications, providing voice and limited internet access.

To address the user requirements identified through e-Navigation, a suite of digital communications options can be used.  VDES provides digital communications capabilities for: addressed unicast (point to point); addressed multicast (point to predetermined group of vessels) and broadcast.  Table 3 presents different existing or developing digital communications technologies.

| Communication Technology | Data rate | Infrastructure | Coverage | Transmission | Maritime / public |
| --- | --- | --- | --- | --- | --- |
| NAVDAT | 12-18 kbps | Based on NAVTEX | 250/300NM | Broadcast | Maritime |
| VDES VDE | 307 kbps | VHF Data link, RR Appendix 18 channels | Line of sight, approx 15NM-65NM Satellite component provides further coverage | Addressed / broadcast | Maritime |
| VDES ASM | 19.2 kbps | VHF Data link, RR Appendix 18 channels | Line of sight, approx 15NM-65NM | Addressed / broadcast | Maritime |
| Wi-Fi (IEEE 802.11ac) | 1,300 kbps | Routers/Access points | 50m | Addressed | Public |
| Digital VHF | 9.6 – 19.2 kbps | Base station/mobile radios | Line of sight, approx 15NM-65NM | Addressed | Maritime |
| Digital HF | 19.2 kbps | Base station/mobile radios | Global | Addressed | Maritime |
| IMT: 4G (including LTE) | 600 Mbps | 4G Base stations | 5-30km (3-6 NM) | Addressed | Public |
| IMT: 5G | 1,200 Mbps | 5G base stations | 5-30km (3-6 NM) | Addressed | Public |
| GEO Satellite |  |  |  |  |  |
| Inmarsat C | 600 bps | Satellite service | Global, spot beams | Addressed / broadcast | Maritime |
| Inmarsat GX | 50 Mbps | Satellite functioning on Ka band | Global, spot beams | Addressed / broadcast | Cross Industry |
| LEO Satellite |  |  |  |  |  |
| Iridium | Up to 134 kbps | Satellite functioning on L band | Global, dependent on constellation size | Addressed / broadcast | Cross Industry (Iridium Pilot Maritime) |

Comparison of Digital Communications Technologies

# POTENTIAL USES OF VDES

To assist in identifying possible options for use of VDES a number of potential scenarios have been developed. These are presented to provide context for development and implementation of digital communications, including VDES. The potential uses of VDES presented here are not intended to be an exhaustive list.  It is expected that VDES uses will evolve as the system is implemented. The use cases are cross referenced to Maritime Service in the context of e-Navigation as noted in Table 4.

| Potential uses of VDES | MS in the context of e-Navigation Reference |
| --- | --- |
| SAR Communications | MS 9 - Telemedical Maritime Assistance Service (TMAS) MS 16 - Search and Rescue (SAR) Service |
| Maritime Safety Information | MS 5 - Maritime Safety Information (MSI) service MS 13 - Ice navigation service MS 14 - Meteorological information service MS 15 - Real-time hydrographic and environmental information services |
| Ship Reporting | MS 8 - Vessel shore reporting MS 15 - Real-time hydrographic and environmental information services |
| Vessel Traffic Services | MS 1 - VTS Information Service (INS) MS 2 - Navigation Assistance Service (NAS) MS 3 - Traffic Organization Service (TOS); MS 4 - Local Port Service (LPS) MS 6 - Pilotage service MS 7 - Tugs service |
| Charts and Publications | MS 11 - Nautical chart service MS 12 - Nautical publications service MS 15 - Real-time hydrographic and environmental information services MS 17 – AtoN service (including PNT and SBAS) |
| Route Exchange | MS 1 - VTS Information Service (INS) MS 2 - Navigation Assistance Service (NAS) MS 3 - Traffic Organization Service (TOS); MS 4 - Local Port Service (LPS) MS 5 - Maritime Safety Information (MSI) service MS 6 - Pilotage service MS 7 - Tugs service MS 8 - Vessel shore reporting MS 10 - Maritime Assistance Service (MAS) MS 11 - Nautical chart service MS 12 - Nautical publications service MS 13 - Ice navigation service MS 14 - Meteorological information service MS 15 - Real-time hydrographic and environmental information services MS 16 - Search and Rescue (SAR) Service |
| Logistics | MS 7 - Tugs service |

Potential VDES Uses cross-referenced to IMO SIP MS update 1

VDES is intended to support the following use cases by terrestrial and satellite communication. The VDES extend the capabilities of AIS.

## Integrity check / authentication for position reports

### Scenario – Authentication for position reports

### Positioning reports are broadcasted without authentication by ships and ATONs through AIS. Without authentication there is no way for the recipient to verify the content of the positioning report. VDE-TER may complement the continuous broadcast stream of AIS positioning reports to authenticate the content through the TESLA protocol considering the requirements of VDE-TER.

### Scenario – Fishing vessels certification

Independent authentication of the location of a fishing vessel with time will be used for the fishery certification to prevent IUU (illegal, unreported and unregulated) fisheries. Consistency between R-mode and GPS location will be an endorsement

### Scenario – Authentication

VDES comprises of AIS, ASM and VDE-TER with different data rate capabilities. Therefore, AIS and ASM have no or very limited capabilities to authenticate the data for the receiver. The authentication of the AIS and ASM messages allows the necessary trust in the shared messages to make safe decisions. VDE-TER could extend repeated or streamed messages from AIS and ASM to provide additional authentication information e.g. by the TESLA protocol.

## Positioning, Navigation and TIming (PNT)

### Scenario - GNSS spoofing/jamming detection

### Scenario - R-mode

### Scenario - Resilient GNSS

- Multi constellation / multiband GNSS

- Multi system GNSS receiver IMO RESOLUTION MSC.401(95) (Adopted on 8 June 2015) PERFORMANCE STANDARDS FOR MULTI-SYSTEM SHIPBORNE RADIONAVIGATION RECEIVERS (GNSS augmentation

- SBAS/GBAS

- Maritime RAIM needs to be updated

### Scenario - Sharing Almanac DGNSS

DGPS stations, eLORAN, AIS message 17 stations, RACON’s….

## SAR communications

SAR Communications are defined in existing documentation (ref SOLAS IV, SAR 79, IAMSAR Manual, NAVTEX manual and SafetyNet manual).

VDES is a technology that supplements AIS communications, and as such may be used for data communication of Maritime Safety Information (MSI) and supplementary distress communications.  VDES supports both addressed (unicast and multicast) and broadcast communications to support SAR response.

When available, the VDES satellite component (VDE-SAT) may be an effective means to extend the VDES to areas outside of coastal VHF coverage.  The VDES-SAT may deliver information in a broadcast, multicast or unicast mode to a broad area, addressing many ships using only minimal radio spectrum resources.  The VDE-SAT will provide a communication channel that is complementary to GMDSS and the terrestrial components of the VDES system (i.e. coordinated with terrestrial VDES, application specific messages (ASM) and AIS functionalities and their supporting systems).

As a communications medium, VDES may be used to relay distress alerts and locating signals (i.e. SARTs).  VDES has also potential to supplement other GMDSS functional requirements which require further development through the GMDSS review process.

In this use case the mix of current communications and developing communications techniques can enhance and improve the sharing of information in prosecution of a SAR incident.  This would include text in free form / standard formats; transfer of waypoints / route information for display on on-board equipment; transfer of GIS information / search patterns; images; etc.

The VDES can be used in SAR planning, execution and decision making.

### Scenario - Distress Communications – Distress Relay

Once a distress alert is initiated and the information has been forwarded to a Rescue Coordination Centre (RCC) through the established GMDSS process, the RCC forwards information of the incident to vessels in the area.  The forwarding of information, using existing formats, could be provided by various communications means, including VDES.  The forwarding of data over a digital communications system such as VDES could facilitate the integration and display of information on external systems on-board (for example, Radar, ECDIS). Information could then be passed to the RCC and other vessels in the area including course to intercept, ETA on-scene, on-scene conditions, sharing of a common operating picture, etc.

### Scenario - SAR Operations – initiate search / response

SAR Mission Co-ordinator (SMC) develops response to SAR using resources, search plan, etc.  Information to prosecute SAR operation is transmitted to the On Scene Commander (OSC) and SAR response units (SRU) (for example information on resources, plan, waypoints for search pattern, SRU responsibilities, etc.).  This could be done using VDES, providing a common operating picture and information using standard templates and formats.

### Scenario - SAR Operations – information exchange

During a SAR mission, the OSC and SRU provide regular updates on the search / response to the SMC.  In addition, the OSC and SRU share information between each other to facilitate the response.  The VDES could be used to exchange information on the SAR plan, SAR execution and other pertinent information to facilitate SAR operations.  Information could be automatically integrated with, and portrayed on, external systems both ashore and on-board, including the RCC GIS, decision planning and support systems.

Using information from other systems, such as vessel route, information provided could be tailored to be relevant for the vessel based on its route.

In addition, the OSC and SRU could share information between each other to facilitate the response.

### Scenario - Tele-medical

A tele-medical happens when a person is injured or sick on-board a vessel or platform and there is a need to communicate with a doctor ashore for medical assistance and prognosis.  The conversation with doctor could be by voice, with transfer of images / photos / indication from medical equipment on patient’s condition.  VDES could be used to transfer advice, images or other information.  Where there are language difficulties, VDES could assist with machine to machine communications and/or language independent communication.  Information exchange could be integrated with, and portrayed on, external systems on-board or ashore (medical facility).

### Scenario - Medevac

A MEDEVAC may be necessary to evacuate a severely injured or sick person.  VDES may be used to exchange pertinent medical information from the ship to the SRU and destination medical facility.  The SAR Mission Co-ordinator (SMC) develops the response to prosecute the MEDEVAC and could use VDES to provide the plan to the ship and responding unit.  Information on the status of the patient could be transferred during the MEDEVAC, both voice and from medical equipment on the SRU.

## Safety Related Information

Information regarding safety of navigation and protection of the environment can be transmitted through the VDES.  Safety related information could use the broadcast aspect of VDES.

This includes Maritime Safety Information (MSI) as defined in IMO SOLAS V, regulation 4 (navigational warnings), SOLAS V, regulation 5 (meteorological services and warnings) SOLAS V, regulation 9 (hydrographic services) and SOLAS V, regulation 31 (Danger Messages).  Other references include MSC.1/Circ.1287 rev1; MSC.1/Circ.1288; (additional reference COMSAR Cir.15) the IMO Worldwide Radionavigation System IMO resolution A.706(17) (as amended) sets out the Worldwide Radionavigation System (WWRNS).

Near real-time information on meteorological and hydrographical information may be provided.

In this use case information shall be transmitted in standardised formats that can take advantage of the VDES capabilities (for example - IHO S-124 formats).

The requirement includes the ability to send information to a predetermined area (i.e. NAVAREA and METAREA) or an area of particular interest defined by administration.

### Scenario - Meteorological Services and Warnings / Navigational Warnings

It is important to have up to date information on the weather that can be expected along a ship’s planned route.  VDES could be used to facilitate information exchange relating to the route of the vessel, integrated with, and portrayed on external systems on-board.

### Scenario - Weather Observations

Ships may participate in the provision of weather observations, as noted in MSC.1 Circ. 1293.  This is a voluntary observing ship (VOS) scheme with information provided to the World Meteorological Organisation.  VDES could facilitate this reporting, with information provided directly from on-board sensors.

### Scenario - Ice maps

Information on sea ice conditions around a vessel is important to help ensure safe passage at sea. Knowledge of areas with sea ice along a ship’s planned route allows ships to find the most efficient route at an early stage. Together with prognoses for expected ice movements, ice charts allow mariners to plan ahead and significantly reduce the risk of vessels becoming ice locked.  VDES could be used to provide this information, which could then be integrated with, and portrayed on external systems on-board.  In addition, information on the latest version of ice maps may be provided from mobile station to mobile station.

### Scenario - Notices to Mariners

Notices to mariners are a means to disseminate navigational safety information (as part of maritime safety information).  SOLAS V, Regulation 9 (Hydrographic Services) notes that administrations should undertake to arrange the dissemination and update of all nautical information necessary for safe navigation (for example. predictive and real-time tides and currents).  VDES could be used to provide this information, and changes to information, with respect to the waterway.

### Scenario - GNSS Augmentation

SOLAS Chapter V, Regulation 19 notes that all ships, irrespective of size shall have a receiver for a global navigation satellite system or a terrestrial radio navigation system, or other means, suitable for use at all times throughout the intended voyage to establish and update the ship's position by automatic means.  IMO Resolutions A.915(22) and A.953(23) provide the requirements for Maritime Radio navigation Systems.  Distribution of GNSS augmentation corrections via VDES could allow GNSS users to get timing, integrity data and improved position accuracy. Research has shown that also timing and radio navigation is also possible by using the VDES. At the moment this is not implemented in the Radio Regulations (RR) but will be a agenda item at the WRC-23.

### Scenario - Crowd sourced information

Information from users or ship systems may enhance and/or validate meteorological hydrological and hydrographic information that is made available to other vessels in the area and authorities.  VDES could be used to facilitate crowd sourced information.

## Ship Reporting

Ship reporting can include mandatory and voluntary reports required for a number of purposes by vessels to various shore authorities.  Information on ship reporting is provided in IMO SOLAS V, regulation 11 (ship reporting systems), 19-1 (LRIT), regulation 31 (danger messages), regulation 32 (information required in danger messages), MARPOL and SAR Convention, Chapter 5.  Additional information on ship reporting is contained in Resolution A.851(20) and FAL.5/Circ.36.

Information forwarded through VDES may transfer data for integration into national and/or regional systems could be sent by VDES to a Maritime Single Window Platform (MSW). (e.g. SafeSeaNet, VTS).  Information may also be sent to the ship agent or owner or a service provider.  Ship reporting could use the addressed (unicast and multicast) aspect of VDES.

### Scenario - Submit arrival notice

A notice of arrival report is based on known content and could be set in a template form.  The aspects of the template report, such as information on the ship particulars, would be pre-populated.  Where possible, additional information related to the voyage, such as destination, ETA destination, last port(s) could be populated from other systems that contain such information.  Other information that may be provided include ISPS reports; ship crew information; information specifically required by the shore authority.  This may be submitted using data populated automatically from other systems or may require manual input by the mariner.  VDES could facilitate exchange of information.

### Scenario - Submit updated information

As the voyage continues, updated information will be provided.  This can include updated estimated time of arrival; change in condition of the vessel; change in route of the vessel.  This is a user defined report, which could be based on a set template for ‘updated information’ or free-text report.  VDES could facilitate exchange of information.  The vessel may be interrogated for information on request, based on its route, operating area or position.

### Scenario - Provide initial report to shore (prior to departure)

Prior to departure specific, standard information is required.  This may include information required for clearance to depart.  Reports could be pre-populated from available information where possible.  Specific information may be required to be entered manually.

### Scenario - Secure ship reporting

There can be times when it is necessary for secure ship reporting, including times when the vessel may disable  AIS transmissions.  Using VDES, information could be forwarded through a secure communications link.

### Scenario - Danger Message

The Master of a vessel is required to report dangerous conditions (SOLAS V, regulation 31 and 32) as well as any irregular conditions referring general marine domain awareness, such as: dangerous ice, derelicts, dangers to navigation, tropical storm, severe weather, ice accretion.  VDES could facilitate the provision of this information to both shore authorities and other vessels in the area.  Information exchange may be integrated with, and portrayed on, external systems on-board.

### Scenario – Reporting on depth/height on current position

Real-time monitoring of under kiel clearance (three dimensional position would be nice – reporting requirement)

Where does the information come from competent waterway authority or ship? What about responsibility?

In some cases data is needed about the depth of a fairway combining the position of the antenna with SBAS accuracy and the depth (echo sounder) of the ship and the difference between the antenna and the echo sounder. (Height of the antenna to sea bottom)

Bridge clearance is depending on the height of the ship and the water level at the moment the ship is planned to cross the bridge or at the dock with cranes.

- Sounder depth

- Height of antenna

- Loaded-unloaded (and effect on ship – draught will change)

- Tide problems

- Difference between antenna position and sounder

- Drawing?

- Structure above receiving antenna

- Resolution A.915(22) Adopted on 29 November 2001 (Agenda item 9) REVISED MARITIME POLICY AND REQUIREMENTS FOR A FUTURE GLOBAL NAVIGATION SATELLITE SYSTEM (GNSS)

### Scenario – Ship Weather observation report from ship

See IMO circ 289 DAC 1 FI 21 lacs satellite link VDES can give this!

The WMO Weather observation report from ship message is intended for ships which have been recruited by national meteorological services to undertake weather observations at sea in accordance with the provisions of SOLAS chapter V, regulation 5, and the World Meteorological Organization's Voluntary Observing Ship (VOS) Scheme. Because national meteorological services are the intended primary users of this message it has been developed to reflect the coding principles prescribed by WMO in its Binary Universal Form for the Representation of meteorological data (BUFR), and as contained in Part B of WMO Publication No.306, (Manual Codes, Volume I.2). The parameters coded in this message are therefore not fully compatible with the recommendations set out in ITU M.1371-3.

## Vessel Traffic Services

Vessel Traffic Services is included in SOLAS Chapter V, Regulation 12, with further information in IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services.

IMO Resolution A.1158(32) states that:

“The purpose of VTS is to contribute to safety of life at sea, improve the safety and efficiency of navigation and support the protection of the environment within the VTS area by mitigating the development of unsafe situations through:

- providing timely and relevant information on factors that may influence ship movements and assist onboard decision-making. This may include:.

  - position, identity, intention and movements of ships; .

  - maritime safety information;

  - limitations of ships in the VTS area that may impose restrictions on the navigation of other ships (e.g. manoeuvrability), or any other potential hindrances;

  - other information such as reporting formalities and International Ship and Port Facility Security Code (ISPS Code) details; and

  - support for, and cooperation with, allied services;

- monitoring and managing ship traffic to ensure the safety and efficiency of ship movements. This may include:.

  - planning ship movements in advance;

  - organizing ships under way;

  - organizing space allocation;

  - establishing a system of traffic clearances;

  - establishing a system of voyage or passage plans;

  - providing route advice; and

  - ensuring compliance with and enforcement of regulatory provisions for which they are empowered,

- Responding to developing unsafe situations, which may include:

  - a ship unsure of its route or position;

  - a ship deviating from the route;

  - a ship requiring guidance to an anchoring position;

  - a ship that has defects or deficiencies, such as navigation or manoeuvring equipment failure;

  - severe meteorological conditions (e.g. low visibility, strong winds);

  - a ship at risk of grounding or collision; and

  - emergency response or support for emergency services.

To achieve their purpose, VTS should provide information or issue advice, warnings and instructions, as deemed necessary”.

The IALA Guideline G1089 “Provision of Vessel Traffic Services (VTS)” has the purpose to provide provide guidance for the provision of VTS to participating ships in a harmonized manner, in accordance with IMO Resolution A.1158(32) and IALA standards..

Information required by the VTS can be both standardised (supported by templates) or specific to a situation.  VTS could use both the addressed  and broadcast aspects of VDES.

VTS involves maintaining a vessel traffic image, and relies on vessel tracking from sensors such as radar, AIS, CCTV, other VTS centres.  The vessel traffic image may be supplement with crowd-sourced information from vessels data (sensor data from ships provided to the shore to expand the traffic image range).

VTS also requires interaction with traffic to respond to developing traffic situations.

VTS relies on the ability to provide essential and timely information; monitor the actions of vessels in the VTS area, including monitoring routes and changes in route; interacting with other VTS centres in the region; interact with other port agencies (allied services).

Ports may also provide a specific Local Port Service (LPS) where it is deemed through a risk assessment that a VTS is not required.

### Scenario - Waterway Monitoring

VTS provides monitoring and other services.  VDES may be used to monitor vessels and autonomously provide information to these vessels based on predetermined parameters as defined by the shore authority.  In addition, VDES may enable sharing of information on synthetic VTS targets from the VTS to vessels transiting the VTS area.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario – VTS Timely and Relevant Information Service

VTS Timely and Relevant Information Service is provided by broadcasting information at fixed times and intervals or when deemed necessary by the VTS, or at the request of a vessel. .  The information provided may include safety information as previously defined.  Additional information could include specific limitations for navigation in the VTS area (for example manoeuvrability limitations; draft restrictions; channel closures; diving operations).  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board. For example, timely information exchange to achieve mutually coordinated shipping will reduce potential risk. Computer-based automated notification between ships is useful.

### Scenario - VTS Responding Service

The VTS Responding Service is described by IMO as ‘a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area’.  The purpose of the VTS Responding Service is to prevent hazardous situations from developing and to ensure safe and efficient navigation through the VTS area.  VDES could be used to exchange this information (e.g. a suggested route).  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario - VTS Monitoring and Management Service

The VTS Monitoring and Management Service is described by IMO as ‘a service to assist on-board navigational decision-making and to monitor its effects’.  VTS Monitoring and Management Service may be provided on request by a vessel in circumstances such as equipment failure or navigational unfamiliarity.  VDES could be used in the exchange of information during the provision of VTS Monitoring and Management Service.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

## Charts and Publications

IMO SOLAS Chapter V, Regulation 27 (nautical charts and nautical publications) notes that charts and publications necessary for the intended voyage shall be adequate and up to date.  Provision of information on charts and publications could use the addressed and broadcast aspects of VDES.

The aim of nautical chart and publication services are to safeguard navigation at sea by providing information such as nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation.

The nautical chart and publication services ensure the official distribution, update and licensing of electronic charts and publications to vessels and other users.

Nautical publications include list of lights, sailing directions, tide and current tables, etc.

There may be a requirement for a ‘user pay’ aspect for some services (i.e. ENC updates).

### Scenario - Updates linked to a ships’ route

An example of this would be a vessel proceeding to a specific location.  At the time of sailing, the vessel had all required charts and publications for the voyage.  These charts and publications were the most up to date at the time of sailing, however some information may have changed during the voyage.  Updated information could be provided through VDES as the vessel as it continues on its voyage, providing information based on the route of the vessel, and limiting the updates to only that information that has changed.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario - charts and nautical publications dynamic up-to-dateness

- ship would report revision of existing charts and nautical publications

- shore would provide reference to relevant updates.

## Route Exchange

The development of e-navigation has highlighted the opportunity to make effective use of digital data exchange to support safe and efficient vessel movements.  Projects such as Mona Lisa, EfficienSea and ACCSEAS have included the development of route exchange.  (IEC 61174 edition 4 annex S refers.)

Route exchange could enhance safety by providing early indication of deviation from the reported voyage plan which may be due to fatigue, weather conditions, or condition of the vessel (possible malfunction).

The use of route exchange could assist with fleet management, whereby information on routes can be exchanged with shore personnel as well as other vessels in the fleet.

In addition, the use of route exchange could assist with route and speed optimisation based on weather and currents; just in time arrival and traffic congestion.

Route exchange could take advantage of the addressed (unicast and multicast) and broadcast aspects of VDES.

### Scenario - Ship to Ship

This scenario is outside of VTS coverage areas, noting that within a VTS area, route exchange would be through the VTS.  Ship to ship exchange could be sequential i.e. allowing a route to be exchanged between stations via multiple hops i.e. stations in between. Ship to ship route exchange would assist vessels on a transit by predicting when interactions may occur.  VDES could assist in the exchange of digital data to facilitate ship to ship route exchange in order to allow exchange of intentions.

### Scenario - Ship to Shore

To inform shipping and other waterway users of possible hazardous situations shore authorities need information about the intentions of the waterway users, such as their intended route.  Based on this information the authorities could organize traffic and, when needed, recommend other routes/possibilities for a safer passage and also provide information about the waterway.

### Scenario - Shore to Ship

Before ships enter a sea area monitored by a shore authority, information about this area could be provided to assist in a safe and efficient passage.  If the route of the vessel is known, information can be tailored to the route.  The shore authority could link the vessel planned route with other information received, such as cargo, and adjust the information as may be required.  Route plans received from ships can be used for detecting possible traffic congestions and high risk situations in advance.  Shore authorities can also send alternative route recommendations to ships when needed.  This allows ships to choose the route that is most suitable for its navigation.  VDES can facilitate reception of route plans which may be integrated with, and portrayed on, external systems ashore.

### Scenario - Navigational Disruption

There may be some event or circumstance that impacts the normal operation of the waterway requiring urgent traffic management to ensure continuity of operations.  VDES could be used to share information on the circumstance, and proposed alternate routing could assist in effective movement of vessels throughout the incident.

### Scenario – High priority ship

In some areas there are ships that have a high priority and should have a “free sailing” in a specific time slot over a specific route. Examples are tide/tidal ships and ships with special cargo.

## Logistics / Services

When sailing from berth to berth before, during and after the voyage there are several logistical aspects that must be addressed.  Most of these are done by an agent on the shore but are changed because of different reasons.  The means of communicating these logistical aspects would depend on the location of the ship / shore elements involved and could include VDES.  In cases where cargo is transferred at sea (transhipment) the location could be out of range of other communications and VDES would be the preferred communication exchange platform.

Information transfer could assist with efficiency of the overall cargo chain (for example the MonaLisa Sea Traffic Management (STM) concept).  Logistics/Services could use the addressed (unicast and multicast) aspects of VDES.

Sharing of route information could assist with allied services related to shipping and ship movements - this could include locks, pilotage, tug allocation, shore resources, and other logistical aspects.  This use case represents the business to business aspect.

Logistical elements where VDES may be an appropriate communication method include:

Transfer of vessel loading plan;

Tug operations;

Pilotage operations;

Stores / supplies / ship bunkers required; waste removal.

### Scenario - Logistic services – ship to shore

Ship arriving at a port forwards revised time of arrival and requests confirmation for stores, fuel, access to waste facilities.  Vessel also requires information on pilots and tugs and other aspects for the transit.  VDES provides an opportunity for automated exchange of information to support these types of requests.

### Scenario - Logistic services – shore to ship

As the ship arrives, the shore authority will provide confirmation regarding offloading, loading of the vessel and respond to requests from the ship.  VDES provides an opportunity for automated exchange of information to support these types of requests.

# DEFINITIONS

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print.  Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

# ACRONYMS

| ACCSEAS | Accessibility for Shipping, Efficiency, Advantages and Sustainability (EU research project) |
| --- | --- |
| ACK | Acknowledgement |
| ACM | Address Complete Message |
| AIS | Automatic Identification System |
| AIS 1 | AIS Default Channel 1 - 161.975 MHz (Ch. 87B//2087) |
| AIS 2 | AIS Default Channel 2 - 162.025 MHz (Ch. 88B/2088) |
| App | Application |
| ASC | Assignment Channel |
| ASM | Application Specific Messages |
| BBM | Broadcast Binary Messages |
| BITE | Built in test equipment |
| CCTV | Closed-Circuit Television |
| Circ. | Circular (IMO document) |
| COMSAR | Sub-Committee on Communications and Search and Rescue (IMO) |
| DSC | Digital Selective Calling |
| ECDIS | Electronic Chart display & Information System |
| ENC | Electronic Navigation Chart |
| ETA | Estimated Time of Arrival |
| FAL | Facilitation Committee (IMO) |
| FEC | Forward error correction |
| GIS | Geographic Information System |
| GMDSS | Global Maritime Distress and Safety System |
| GMSK | Gaussian Minimum Shift Keying |
| GNSS | Global Navigation Satellite System |
| IAMSAR | International Aeronautical and Maritime Search and Rescue (manual) |
| IEC | International Electrotechnical Commission |
| IHO | International Hydrographic Organization |
| IMO | International Maritime Organization (UN) |
| INS | Information Service |
| ISPS | International Ship and Port Facility Security (Code) |
| ITU | International Telecommunication Union |
| ITU-R | International Telecommunication Union-Radiocommunication Sector |
| kbps | kilobits per second |
| kHz | Kilohertz |
| LPS | Local Port Service |
| LRIT | Long Range Identification & Tracking |
| MARPOL | International Convention for the prevention of pollution from ships (IMO 1973 as amended) |
| MAS | Maritime Assistance Service |
| MCS | Modulation and Coding Scheme |
| MEDEVAC | Medical evacuation |
| METAREA | Geographical sea region for the purpose of co-ordinating the transmission of meteorological information |
| MHz | Megahertz |
| MMSI | Maritime Mobile Service Identity |
| MSC | Maritime Safety Committee (IMO) |
| MSI | Maritime Safety Information |
| MSP | Maritime Service Portfolio(s) |
| NACK | Not Acknowledgement |
| NAS | Navigation Assistance Service |
| NAVAREA | Geographic area in which various governments are responsible for navigation and weather warnings |
| NAVTEX | Navigational Telex (service) |
| NCSR | Navigation, Communications and Search and Rescue (IMO Sub-Committee) |
| OSC | On-scene commander |
| PI | Presentation Interface |
| PSK | Phase-Shift Keying |
| QAM | Quadrature Amplitude Modulation |
| QPSK | Quadrature Phase-Shift Keying |
| RCC | Rescue Co-ordination Centre |
| RA | Random Access |
| RACH | Random Access Channel |
| RF | Radio Frequency |
| RR | Radio Regulations |
| RTCM | Radio Technical Commission for Maritime Service |
| SAR | Search and Rescue |
| SART | Search and Rescue Transponder |
| SAT | Satellite |
| Serv | Server |
| SIP | Strategic Implementation Plan |
| SOLAS | International Convention for the Safety of Life at Sea, (IMO 1974 as amended) |
| SRU | Search and Rescue Unit |
| TBB | Terrestrial Bulletin Board |
| TDMA | Time-division multiple access |
| TMAS | Telemedical Maritime Assistance Service |
| TOS | Traffic Organization Service |
| UDCH | User Data Channel |
| UTC | Co-ordinated Universal Time |
| VDE | VHF Data Exchange |
| VDES | VHF Data Exchange System |
| VDL | VHF Data Link |
| VHF | Very High Frequency (30 MHz to 300 MHz) |
| VOS | Voluntary observing ship |
| VTS | Vessel Traffic Service |
| WRC | World Radiocommunications Conference |
| WWRNS | World-wide Radionavigation System |

Technical Overview of VDES

Introduction:

The VHF Data Exchange System (VDES) extends the Automatic Identification System (AIS) as defined in the ITU-R M.1371 by adding new technology.  VDES is defined in ITU-R M.2092.

This technical overview assumes a basic knowledge of AIS and provides an overview of VDES.

VDES description:

VDES is a combination of technologies that includes AIS, ASM and VDE. VDE consists of a terrestrial (VDE-TER) and a satellite (VDE-SAT) part. The VDES Presentation Interface (PI) uses NMEA sentences, and is similarto  the AIS PI but will include several new IEC 61162-1 sentences to allow for the configuration of the VDES capability and facilitate the transfer of larger amounts of data.

The primary differences between AIS and the two new VDES technologies, ASM and VDE, are:

- New modern Modulation and Coding Scheme (MCS).

- New dedicated Radio Frequencies (RF) used the Radio Frequency bandwidth.

- Higher data bandwidth.

- New methods used by the Link Layer.

- ASM MCS is designed to improve reception on low-earth orbit satellite.

- VDE includes dedicated MCS and Link Layer for satellite uplink and downlink (VDE-SAT)

VDE has been designed to efficiently transfer relatively large volumes of arbitrary data. Unlike AIS, the definitions of the data being transferred is not defined by VDE. VDE only defines the data transfer mechanism. It can provide data transfer up to 300x higher when compared to AIS.

VDE has been designed to co-exist with AIS as to not interfere with AIS.

VDE-ASM is intended to move ASM traffic away from the existing AIS channels in order to improve safety at sea.

The Modulation and Coding Schemes

The modulation and coding schemes range from Gaussian Minimum Shift Keying (GMSK) for AIS to π/4 Quadrature Phase-Shift Keying (π/4QPSK) for ASM and π/4QPSK and 16 Quadrature Amplitude Modulation (16QAM) for VDE.

The Radio Frequency (RF) bandwidth on the technologies differ with AIS and ASM both having a RF bandwidth of 25kHz per channel and VDE having a bandwidth of 25kHz, 50kHz or 100kHz per channel.

AIS uses AIS1 and AIS2; ASM uses ASM1 and ASM2; and VDE uses VDE1a and VDE1b (refer to figure 2).

AIS and ASM are simplex channels (can carry traffic in one direction at a time) and VDE1a and VDE1b form a duplex pair that allows data to be transferred in both directions at the same time.

The radio frequencies used and bandwidth

At an overview level, the AIS and ASM use 2 X 25kHz channels. VDE can use 2 X 25kHz; or 2 X 50 kHz or 2 X 100kHz channels in the maritime VHF band. The allocated frequencies are as follows:

| Technology | Radio Frequencies used | Radio Regulations channel number |
| --- | --- | --- |
| AIS 1 | 161.975 (25kHz) | 87B |
| AIS 2 | 162.025 (25kHz) | 88B |
| AIS Long Range 1 | 156.775 (25kHz) (ships are TX only) | 75 |
| AIS Long range 2 | 156.825 (25kHz) (ships are TX only) | 76 |
| ASM 1 | 161.950 (25kHz) | 2027 |
| ASM 2 | 162.000 (25kHz) | 2028 |
| VDE 1 | 157.200 to 157.275 (100kHz) (Ship TX) | 1024, 1084, 1025 and 1085 combined |
| VDE 2 | 161.800 to 161.875 (100kHz) (Ship RX) | 2024, 2084, 2025 and 2085 combined |

Frequencies allocated to VDES

The data bandwidth

The modulation schemes π/4QPSK (ASM and VDE), 8PSK and 16QAM (VDE only) allow more data to be transferred in the same Radio Frequency (RF) bandwidth than was possible with AIS (GMSK).

The increase in data bandwidth is illustrated in Figure 5.

Increasing data bandwidth in VDES

Note: Figures shown reflect raw bandwidth. It is expected that there will be up 33.4 kbps (net bitrate) per vessel.

## The methods used by the Link Layer

The ASM and VDE technologies use the same AIS slot map of 2,250 slots per RF channel (4,500 slots for two RF channels used in parallel in AIS and ASM technologies).  Due to the larger data bandwidth resulting from the use of π/4QPSK modulation, the number of bits transferred in any one slot is increased in ASM when compared with AIS. VDE-TER can transfer up to 210.6 kbps (LCI 19), and VDE-SAT has a maximum transfer rate of 47.75 kbps (downlink) and 94.70 kbps (uplink). The VDE frequencies, Modulation and Coding Scheme (MCS) are dynamic and are chosen by a combination of the Terrestrial Bulletin Board (TBB) (section A 4 refers) and by the Link Layer.

The Link Layers of AIS, ASM and VDE are different and each are optimised for the data bandwidth available.

The VDE Link Layer is more complex and uses a different Link Layer to facilitate communication between vessels (ship to ship) and between the shore and a vessel (shore to ship).

Forward Error Correction

ASM and VDE uses Forward Error Correction (FEC) which allows for the correction of errors in the data transferred in both ASM and VDE. FEC increases the integrity and reliability of the transmission.

The Terrestrial Bulletin Board

The VDE technology uses a Terrestrial Bulletin Board (TBB) to assign the primary operating environment parameters to the Control Station Service Area. This includes which frequencies are being used and the service area dimensions amongst a range of other technical detail. To provide a level of protection to the VDES communication environment, the TBB may be authenticated.  Authentication confirms that the TBB is transmitted by a trusted entity.

VDE Channel resources for data transfers are managed by the Control Station. Ships are within a Control Station service area when they receive a valid TBB that also overlaps their position.

Logical channels

VDE Channel ressources are managed as logical channels. One of 5 logical channels are dynamically allocated (shown as logical channel 8, 9, 10, 11 and 12 in the diagram below) to carry the data in the User Data Channel (UDCH). The TBB (TBB), Assignment Channel (ASC) and Random Access Channel (RACH) are defined too ensure that the VDE channel is optimally used to carry user data.  Figure 6 provides an example of channel configuration.

A mobile station requests access to a logical channel from the Control Station when it needs to transfer data over VDE. The logical channels have been designed in such a way that a VDES terminal can provide both VDE and AIS functionality without affecting AIS. Multiple terminals can simultaneously transfer using a TDMA access scheme that makes optimum use of the available resources.

The Random Access Channel are used to request resources and can also be used for small data messages allowing the User Data Channels to be available for larger messages.

The User Data Channel is allocated to the mobile station for  a fixed period. The user can continue to request the resource for as long as it has data to transmit. This is done in an optimal way on the Data Channel without having to use the Random Access Channel again. The Control Station determines if the request is granted or not.

The VDE control station transmits the TBB at the start of a frame. Just as in AIS, the VDE frame is one minute in duration comprised of 2250 slots.

When a mobile station is outside the service area of a control station, the ship can transmit data directly to another ship using ship to ship mode.  A ship it is out of the service area when it does not receive a valid TBB from a VDE Control Station for 15 minutes or the vessel is in a position outside of the service area of any received TBB.

VDES Ship and Shore station

A VDES mobile station consists of (at least) 6 radio receivers.  There are 2 AIS receivers, 2 ASM receivers and 6 or more VDE receivers.  AIS, ASM and VDE all share the same transmitter. Every transmitter and receiver is connected to the VDES controller that manages the radio protocol. The VDES controller takes its input from the PI, and outputs received radio messages from all the receivers on the same interface.  GNSS for positioning and timing is done in the same way as for AIS. A typical block diagram of a VDES mobile station is shown in [Figure 7].

VDES ship station logical description

A VDES control station consists of the same logical components as the mobile station, but has differences in the PI, Link Layer functionality and the antenna system (the control station allows for connection to a more complex antenna system by separating the receive and transmit paths to the antenna system). A shore station will always allocate resources within its service area and a ship must always request it from the shore station when it falls within its service area. A typical block diagram of a VDES control station is shown in [Figure 8].

VDES shore station logical description

The VDES fit into the Common Shore Side Architecture (IALA Guideline 1113) in the same manner as does AIS as illustrated below.

VDES into the Common Shore Side Architecture

Example transaction between ship and shore

The transfer starts with the mobile station requesting VDE Control Station resources. The VDE Control Station allocates an exclusive Logical Channel for the transfer. The mobile station sends then multiple data fragments that are acknowledged by the VDE Control Station. Fragments that are not acknowledged are to be resent.

Ship control station

VDE link layer messages

SatelLite

[Stefan Pielmeier fills more in for eNAV29]

ASM

[Johnny fills more in for eNAV29]

Recommendation ITU-R M.493

Recommendation ITU-R M.1371

Recommendation ITU-R M.1842

More information on VDL loading is available in Report ITU-R M.2287-0

IEC standard 61174 Ed. 4 includes route exchange.